Government

of Tonga

Energy Efficiency Master Plan

June 2020

blank page

# Government of Tonga

# Energy Efficiency Master Plan

Nuku’alofa, Tonga, 2020

copyright page

Contents

Abbreviations	iv

Foreword  	v

Executive summary	1

1	Context	4

1.1	Background and focus of effort	4

1.2	About Tonga	6

2	Baseline	7

2.1	Electricity baseline	8

2.1.1	Generation, transmission and distribution	9

2.1.2	Consumption	12

2.2	Baseline transportation	20

2.2.1	Land transportation fuel usage	20

2.2.2	Vehicle kilometers travelled	21

2.2.3	Vehicles	24

2.2.4	Other baseline data of interest	25

3	Business-as-usual projections	26

3.1	Tonga transportation fuel business-as-usual projection	26

3.2	Tonga electricity business-as-usual projection	28

4	Proposed reduction targets	30

4.1	Transportation wedge analysis	31

4.2	Electricity wedge analysis	33

4.3	Combined wedge analysis	34

4.4	Energy flow diagrams of 2017 baseline and 2030 goal	34

4.5	Energy intensity with TEEMP policy changes	35

5	Projects and policies to reduce GHG emissions	36

5.1	Transportation policies and projects	36

5.2	Electricity energy efficiency policies and projects	41

5.3	Resilience considerations	44

6	Monitoring the TEEMP	46

7	Financing the TEEMP	50

8 Conclusion	56

Appendix A. Supporting documentation	57

Endnotes	59

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

- Abbreviations

BAU		Business as Usual

CO2e		Carbon dioxide equivalent

COP		Conference of Parties

EE		Energy efficiency

GDP		Gross domestic product

GHG		Greenhouse gas

GOT		Government of Tonga

GREET		Greenhouse gases, Regulated Emissions, and Energy Use in Transportation

HDV		Heavy-duty vehicle

INDC		Intended nationally determined contribution

IRP		Integrated Resource Planning

kg		kilogram

kWh		kilowatt hours

LED		light emitting diodes

LDV		light-duty vehicle

MTED 	Ministry of Trade and Economic Development

MWh		megawatt hours

PRDR 		Pacific Regional Data Repository

PV		photovoltaic

TEEMP		Tonga Energy Efficiency Master Plan

T&D		Transmission and distribution

TERM		Tonga Energy Road Map

TPL 		Tonga Power Limited

VKT		vehicle kilometers traveled

W		Watts

- Foreword

It is my pleasure to present herewith Tonga’s Energy Efficiency Master Plan (TEEMP): 2020 – 2030. This document builds on Tonga’s ratification of the Paris Agreement in 2016 and recommends ambitious and urgent actions that will contribute to pursuing targets in Tonga’s Nationally Determined Contribution.

The TEEMP represents the Tonga Government’s commitment to join the global community in addressing the challenges of climate change and improving national energy security. It is a whole-of-government approach to maximising benefits from low hanging fruits in energy efficiency.

The TEEMP is in line with the landmark Tonga energy road map: Ten year road map to
reduce Tonga’s vulnerability to oil price shocks and achieve an increase in quality access to modern energy services in an environmentally sustainable manner. It is also in line with the Tonga Nationally Determined Contribution and its Low Emission Development Strategy.

The TEEMP therefore highlights the direct correlation between renewable energy and energy efficiency. When there is more renewable energy capacity, diesel engines are on standby mode  and issues with the energy efficiency of the diesel engines are avoided.

The production of the TEEMP was through an open and consultative process that sought to incorporate ideas and inputs from a wide spectrum of stakeholders. It is in a similar collaborative spirit that the implementation and review of the progress of the TEEMP will be conducted.

It is a coincidence that we are launching the TEEMP in year in which the globe has been challenged by a pandemic, whose extent and impacts have never before been confronted by humanity. It is a stark reminder of the significance of the energy sector for our very existence. As we are continuously reminded to wash our hands, we are reminded of the need for clean, affordable energy to pump water and make it accessible. Furthermore, this is a time where we are reminded that energy supply must be very reliable in order to keep lifesaving equipment working without interruption.

I commend this master plan and the information it contains, and hope that its impacts will benefit all.

'Ofa atu

Hon. Dr Pohiva Tu’ionetoa
Prime Minister of Tonga
July, 2020

- Executive summary

At the request of the Tonga government, the Climate Technology Centre and Network has worked closely with the Energy Department to develop a Tonga Energy Efficiency Master Plan (TEEMP) for adjustment and adoption by the relevant Tongan entities. The plan is based on a study of existing frameworks, plans, programmes and projects; extensive stakeholder consultations; and data development and analysis. The TEEMP encompasses electricity use and ground transportation. It complements the approach of the 2009 Tonga Energy Road Map 2010–2020 (TERM). The TERM focuses on lowering Tonga’s fossil fuel dependence through increased energy efficiency and improved supply chains to reduce price fluctuation of imported products for the purpose of reducing greenhouse gas (GHG) emissions and improving national energy security.

A baseline assessment of Tonga’s GHG emissions was built from Tonga’s Intended Nationally Determined Contribution (INDC), which identifies the key emitting sectors as transport (40%), electricity generation (23%), agriculture (21%), waste (11%), and other energy (5%). The TEEMP addressed 55% of these total greenhouse gases: electricity generation (23%) and ground transportation (32%). A Sankey analysis determined energy source flows by fuel type for building electricity consumption and transportation. Roughly half of all diesel consumption goes towards transportation and the other towards electricity generation for building electrical use (primarly space cooling, lighting, and appliances in the residential and commercial sectors). The remainder of transportation fuel is petrol.

As of 2017, the total installed capacity on Tonga’s largest island, Tongatapu, was 17.8 megawatts (MW), with 14 MW of conventional capacity from diesel generators, 2.3 MW from photovoltaic (PV), 0.5 MW from wind, and a 1 MW battery storage system. This installed capacity is an increase from 2012, when Tongatapu had 12.6 MW of conventional capacity and 1.3 MW of renewables. The growth in capacity is attributable to government leadership to deploy more renewable energy generation, and increased electrical load, primarily driven by increased ownership of electrical appliances. Tonga Power Limited (TPL), the vertically integrated utility, owns and operates the majority of Tonga’s in-front-of-meter electricity generation, as well as its transmission and distribution (T&D) assets.  Electricity rates have important implications for incentivising or disincentivising energy efficiency investments. Despite differences in generation cost, the TPL tariff rates are standardised across the four main islands and, as of February 2018, the tariff was set at 0.8514 Tongan Pa’anga (TOP) per kilowatt-hour (kWh). There is also a subsidised “lifeline tariff” of 0.7 TOP for the first 100 kWh of monthly consumption, applicable to all customers. Given that most of Tonga’s electricity is generated from diesel, the price of electricity is sensitive to fluctuations in fuel costs.

Tonga’s estimated energy intensity is 216.8 gigajoules (GJ) of imported fuel per dollar (USD) of GDP per capita, rising under the business as usual (BAU) scenario to 259.8 GJ by 2030.  In 2017, the residential sector accounted for 44% of electricity consumption, with commercial, religious, government and public service accounts making up the remaining 56%. The TERM indicates an expected growth in energy consumption of 28% by 2020, compared with 2010 levels. An estimated baseline for transportation is over 16,000 vehicles in Tonga for residential, commercial, and government use in 2016. The majority of vehicles were either cars (6,031) or light trucks/vans/SUVs (7,103). Heavy duty vehicles, taxis, rentals, motorcyles and buses accounted for the remaining 3,690 vehicles. Current average vehicle kilometers travelled per person was estimated to be 2,289 in 2016 and is projected to grow to 5,103 by 2050, tracking with anticipated GDP increases.

Key policy options identified for transportation include vehicle import tariffs or registration fees aimed to improve fuel economy; restrictions in heavy-duty vehicle (HDV) idle time; 10% biodiesel blend; deployment of electric vehicles; and policies that reduce vehicle kilometers travelled by facilitating pedestrians, bikers, carpoolers and bus riders. These reductions in energy use in the transportation sector would result in an estimated 28% reduction in greenhouse gases below business-as-usual levels by 2030, which is an increase of 1% over the 2018 baseline.

In the building sector, Tonga’s electricity consumption is driven by building design, appliance use, and behaviour around energy consumption. Given Tonga’s tropical climate, cooling in buildings in commercial, government and nongovernmental buildings via the use of air conditioning is widespread and increasing. There may be opportunities to reinstitute design practices from historical buildings, such as passive ventilation and large overhangs to provide shade. Leading energy use reduction options in the building sector centre around increased deployment of renewable energy, reductions for implementation of minimum energy performance standards (MEPs), improvements to space cooling, increased efficiency in appliances, and reductions in energy used for lighting (in buildings and street lighting applications).

Taken together, reductions in energy use in both transport and buildings would offer a reduction of 106,000 metric
tonnes of carbon dioxide equivalent (CO2e) per year by 2030, which is a 50.5% reduction from BAU emissions of 210,000 metric tonnes of CO2e. This is based on 2020 as the reference year. As shown in Figure E1, transportation accounts for 30.4% of the reduction, renewable electricity (RE) for 40%, and energy efficiency in buildings and lighting for 29.6% of the reduction. If Tonga reaches 70% RE by 2030, it is assumed there will be only a slight increase in petroleum energy intensity by 2030. But if RE penetrations are greater than 50%, this energy intensity could be reduced, thereby also supporting Tonga’s energy independence and security.

Figure E1. Wedge analysis of Tonga’s GHG emissions

Tables E1 and E2 provide further assessments of the top ten policy options for transport and electricity in buildings and street lighting that could support GHG reductions identified in the wedge analysis (additional policy options and measures are identified in this report). The options are ranked by magnitude of GHG reductions, upfront cost, time to implement, ancillary benefits, and support available in terms of financing and political will. Green indicates positive attributes, yellow neutral, and red negative attributes.

Table E1. Top ten policies and projects to reduce GHG emissions from transportation

Table E2.  Top 11 policies and projects to reduce GHG emissions from electricity

Given Tonga’s vulnerability to climate change and tropical storms, it is important to consider how improvements in the electric and transport sectors can also support increased resilience in terms of enhancing the ability of energy and transportation infrastructure to withstand and recover more quickly from natural and manmade disasters. The primary ways in which Tonga can strengthen its resilience are by:

- incorporating resilience into building codes to enable buildings and structures to better withstand storms, while also supporting energy efficient, passive design;

- continuing to harden T&D lines and power generation by burying strategic lines and installing minigrids and onsite renewable energy, among other measures;

- diversifying the fuel mix from diesel to renewable natural gas and exploring procuring fuel from sources other than Singapore;

- improving vehicle efficiency, which enables more transport options in case fuel supplies are diminished;

- reducing dependence on roads, which may become damaged in a disaster, by enabling additional modes of transport, such as water taxis, and shortcutting large sections of road with a bridge; and

- optimising the use of appropriate vehicles, such as buses with raised clearance, and effectively utilising the bus system to support evacuations.

Context

Tonga is dependent on imported petroleum to meet its energy needs for electricity and transportation. Electricity generation consumes upwards of 13 million litres of fuel per year at a cost equivalent to around 10% of total GDP and transportation consumes 25 million litres at a cost close to 20% of GDP. Dependence on imported fuels places Tonga in a vulnerable position due to volatile fuel prices, which have a downstream effect on electricity, transportation and cost of living expenses. This study provides background information in order to set an energy and emissions baseline and establish a business as usual (BAU) projection. The data collection methods and sources highlighted in this report will also help Tonga track energy use and emissions in the future. The BAU was taken as a starting point to assess options, magnitude and feasibility of reducing Tongan GHG emissions. The Tongan Energy Department requested technical assistance from the United Nation’s Climate Technology Centre and Network (CTCN). Under the CTCN, support was provided for technical assistance and capacity building for the development and implementation of a Tonga Master Energy Efficiency Action Plan. This document hereby represents the flagship deliverable of this effort and reflects extensive research conducted in 2017 and 2018 via in-person stakeholder consultations, policy and regulatory analysis, and modelling.

## Background and focus of effort

In 2009, Tonga developed the Tonga Energy Road Map 2010-2020: Ten Year Road Map to Reduce Tonga’s Vulnerability to Oil Price shocks and Achieve an Increase in Quality Access to Modern Energy Services in an Environmentally Sustainable Manner, known as the TERM. The goal was to create an approach to reduce Tonga’s dependence on fossil fuels through cost-effective and sustainable efforts. The process to develop the TERM was a joint effort of the Government of Tonga, the electricity generator and distributor Tonga Power Limited (TPL), and other development partners. The TERM focuses on electric energy efficiency, improving supply chains to reduce price fluctuation of imported products, reducing greenhouse gas (GHG) emissions and improving national energy security. The TERM establishes a goal of approving a policy to supply 50% of electricity generation through renewable resources by 2012. It also includes improvements in the petroleum supply chain to reduce the price and price fluctuations of imported petroleum products; efficiency of conversion of petroleum to electricity (i.e. increases in efficiency and reduced losses at TPL); efficiency of conversion of electricity into consumer electricity services (demand side management measures); and replacing a portion of current or future grid-based generation with renewable energy. Transportation is not a focus of the TERM.

Policies and initiatives that have been implemented since the creation of the TERM are listed below.

- The National Strategic Planning Framework (2009)

- The Electricity Act (2007)

  - Established an Energy Commission appointed by the Monarch-in-Council, which has broad regulatory authority. The act sets standards for generation, supply and distribution concession contracts submitted to the commission.

- The Renewable Energy Bill (2008)

  - Established a Renewable Energy Authority, consisting of members appointed by the Monarch-in-Council and tasked to develop renewable standards, conduct research, relay international obligations to the monarch, and enforce violations of the renewable energy policy against offenders.

- Joint National Action Plan on Climate Change Adaption and Disaster Risk Management (2010)

  - Assesses geographic risk posed by natural disasters such as cyclones and tsunamis. Provides the framework and financing structure to integrate disaster and risk planning into budgeting at all levels.

- Ozone Layer Protection Act (2010), amended (2014)

  - Bans the importation of products (refrigerators, freezers, dehumidifiers, air conditioners, heat pump units, vehicle air conditioners, ice machines, water coolers, aerosol products, fire extinguishers, insulation boards, and pre-polymers) which contain hydrochlorofluorocarbons (HCFCs), which are ozone-depleting substances and also potent GHGs.

- Tonga Village Network Upgrade Project (2012)

  - Reduced line losses from 18% to 12% in 20 villages, working to expand to more.

- Promoting Energy Efficiency in the Pacific (2014)

  - Funded by the Asian Development Bank, Australia, and Japan.

  - Residential light replacements saving 383,999 kWh/year.

  - Public service building light replacements saving 158,706 kWh/year.

  - Street light upgrades saving 97,219 kWh/year.

- Tonga’s Intended Nationally Determined Contribution (INDC 2015)

  - Formalises 50% renewable electricity goal by 2020, 70% by 2030.

  - Reduce line losses to 9% by 2020.

- Tonga Strategic Development Framework II (2015–2025)

  - Revisits the original TERM goal of 50% renewable from 2012 to 2025; 2020 for rural communities. This contradicts the INDC released the same year.

  - Achieve no more than 12% T&D losses by 2020.

  - Short-term goal to convert 75% diesel-based water pumping systems in Tongatapu to solar photovoltaic (PV).

  - Goal of 80% of retail stores to sell reliable compact fluorescent lightbulbs at “same cost”.

  - Banning import of inefficient electrical appliances.

  - All inefficient electrical appliance banned at border control.

- Outer Island Renewable Energy Project (2017)

  - The first phase of the Tonga Renewable Energy Plan.

  - Project to rebuild ‘Eua’s grid was announced in 2012.

  - Invitation for international bids announced in February 2017.

- Tonga Renewable Energy Plan (2017)

  - A project supported by the Green Climate Fund to finance projects intended to contribute towards Tonga’s 50% renewable by 2020 plan. Total project cost is estimated at USD $84.47 million, with $64.37 million requested from the green climate fund, $9 million from the Government of Tonga, and $6.10 million from TPL.

  - Phase 3 of a broader plan, including the Outer Island Renewable Energy Project (Phase 1), and PPA procurement of solar and wind resources (Phase 2).

  - Includes the installation of a new biomass plant on Tongatapu, along with battery storage, PV and wind.

Figure 1: Map of Tonga
Source: University of Texas

About Tonga

Tonga’s geographic location, demographics and governance structure have important implications for the energy sector. Tonga is in the Pacific Ocean, as shown in Figure 1, and is comprised of 170 islands, 36 of which are populated.

Tonga is prone to both earthquakes and tropical cyclones. Notable instances include a 2009 magnitude 8.1 earthquake and subsequent tsunami, as well as 2014’s Cyclone Ian, which struck Ha’apai directly, damaging or destroying 90% of distribution and transmission lines. In 2018, Cyclone Gita brought the strongest winds Tonga had seen in 60 years, damaging 40–50% of Tongatapu’s electric grid. In the average El Niño year, 1.88 cyclones affect Tonga, 1.63 in La Niña years, and 1.67 during neutral years. Tonga’s government vocally attributes the increasing severity of tropical cyclones to climate change; in response, policy documents focus on planning for resilience.

Due to its small economy and lack of a significant local manufacturing base, Tonga imports almost all its goods, including fuel for energy generation and transportation, vehicles, appliances, and electronics. The TERM recognises the vulnerability this dependence has on Tonga and the adoption of  a renewable energy target of 50% by 2020  to address the challenges created by relying on imported goods and fuels.

Tonga is a consitutional monarchy with a government made up of the executive, legislature and the judiciary. The current monarch  is a champion for sustainable energy in Tonga, with most of the significant renewable energy installations in the kingdom being commissioned during his reign. Both the king and his cabinet are supportive of reducing the effects of climate change on Tonga and improving the nation’s energy independence. In a move symbolic of wider leadership, Tonga joined the Solar Head of State programme at the Conference of Parties 23 in November 2017. The programme will install a rooftop photovoltaic (PV) system on the residence of the king, the first royal palace in the world to have this.

The Department of Energy under the Ministry of Meteorology, Energy, Information, Disaster Management, Environment, Climate Change and Communications (MEIDECC) is responsible for coordinating and managing the energy sector. Other key players in the sector are: Tonga Power Ltd, a public enterprise that is responsible for the generation, transmission and distribuiton of electricity; the Tonga Electricity Commission, which is the regulator in the electricity sector; the Ministry of Trade and Economic Development, which is responsible for the pricing of petroleum products; and the Association of Members and Registered Electricial Contractors (AMREC).  MEIDECC is currently working on an energy act to more effectively coordinate and manage the energy sector.  Data availability has important implications for MEIDECC’s role – analysing and tracking changes and opportunities in the energy sector, and thus for the design of Tonga’s Energy Efficiency Master Plan (TEEMP). Specific recommendations broached later on include engaging TPL in an integrated resource planning process, collecting sectoral data, surveying energy usage habits in future censuses, and standardising data collection of Customs and annual ministerial reporting. Regional initiatives, such as the Pacific Regional Data Repository, already exist to aggregate applicable datasets; working with such initiatives will open opportunities for regional knowledge-sharing and project-financing.

Baseline

As reported in Tonga’s INDC and based on Tonga’s GHG emissions breakdown in the inventory conducted in 2006, Tonga’s major emissions arise from transport (40%), electricity generation (23%), agriculture (21%), waste (11%) and other energy (5%). A baseline study was conducted as part of the TEEMP process to determine where savings could be made within each of the sectors to assist Tonga in achieving goals established in various legislation and policies. Using available data, the breakdown of Tonga’s emissions was estimated, as shown in Figure 2. It is important to note that only 55% of the emissions in Tonga’s overall portfolio are being addressed through the TEEMP: electricity generation (23%) and ground transportation (32%). It does not necessarily mean that the 21% from agriculture is not important to the TEEMP; it was simply a constraint on the data availability and the timeframe for the production of the TEEMP. Hopefully future updates of the TEEMP will include agriculture, waste and the others.

Figure 2: Tonga’s INDC reported emissions and TEEMP-related GHG emissions (Data source: Tonga INDC and the Ministry of Labour and Commerce)

Tonga’s INDC states that transportation is responsible for 40% of emissions, and electricity generation is responsible for 23% of Tonga’s emissions but the INDC used GHG data from a 2006 inventory. Since the INDC was submitted, TPL’s billed customer consumption has risen by approximately 47%, most likely as a result of an increase in electrification efforts, so the proportion of Tonga’s emissions from electricity is assumed to be higher than stated in the INDC. It is assumed that emissions from non-TPL generators are negligible.

The transportation segment of the INDC includes air and marine transport, but the TEEMP addresses only ground transportation, such as light and heavy-duty vehicles. The TEEMP estimates that ground transportation accounts for 80% of Tonga’s total transportation-related emissions, as calculated in the INDC.

Using the available data from the Ministry of Trade and Economic Development (MTED), a Sankey diagram was created to demonstrate the energy flows, by fuel type, for electricity consumption and transportation fuel consumption for Tonga, as shown in Figure 3. These energy flows are influenced by the climate and infrastructure of the islands.

Figure 3: Tonga’s current energy flows by fuel type in the electricity and ground transportation sectors (Data source: MTED)

## Electricity baseline

Census figures report that the 2016 population of Tonga was 100,651. A 2010 survey indicated that 80% of the rural population and 100% of the urban population have access to electricity. In aggregate, the study reported that 92% of the total population had access to electricity.  The 2016 census indicates that 16,662 households get power from the public power supply, 767 from self-owned solar, and 44 from their own generators. TPL saw a 9% growth in new customer accounts between 2016 and 2017. In total, the residential sector accounted for 44% of electricity consumption in 2017. TPL does not further breakdown sectors, but the remaining 55% of consumption is spread across commercial, religious, government and public service accounts. There are two peak demands during weekdays: one in the mornings and another in the late afternoon. Typically, this type of peak load profile indicates commercial customers are using energy (computers, lighting, air conditioning) when they arrive during the morning hours, and a second peak when energy needs shift to the residential sector, when lighting and appliances are used in homes in the evening.

Based on TPL’s 2017 Annual Report, the TEEMP model assumes that the weighted average efficiency of TPL’s diesel generators are 4.15 kWh per litre of diesel fuel. The model allows for generator efficiency to be updated if overhauls or refurbishments are performed in the future. The interaction between renewable energy technologies and diesel generators can also affect the efficiency of generators if they are required to cycle on and off frequently. The installation of battery storage systems, as planned, can help alleviate this. For both the electricity and transportation analysis, data from the Greenhouse Gases, Regulated Emissions, and Energy Use in Transportation (GREET) Emissions Database was used to estimate emissions of diesel fuel at 3.29 kilogram (kg) CO2e / litre. This number is a lifecycle emissions calculation for the United States of America (USA) so it includes emissions associated with extraction and transportation of fuels within and to the USA. Emissions associated with transportation of fuel to Tonga are most likely higher. The TEEMP model also accounts for reductions in line losses; Tonga’s INDC sets the target of 9% by 2020 (currently around 11%). In the model, line losses are applied evenly to all generation (diesel and renewables). In reality, renewable installations are likely to be located closer to the load and therefore will have lower line losses than diesel generation.

In 2009, Tonga’s energy intensity was reported as shown in Figure 4. Through data collected for the TEEMP, Figure 5 shows the 2018 energy intensity, indicating efficiency improvements. This analysis identified an energy intensity of 216.8 GJ of imported fuel per dollar (USD) of GDP per capita, rising under the business as usual (BAU) scenario to 259.8 GJ by 2030.

Figure 4: Tonga’s energy intensity, 2009

Figure 5: Tonga’s energy intensity, 2017–2030 (BAU)

### Generation, transmission and distribution

TPL is a vertically integrated utility responsible for the generation, transmission, and distribution of electricity. It operates four grid systems in Tonga. The largest grid is on the island of Tongatapu, followed in size by the islands of Vava’u, Ha’apai and ‘Eua. The following sections detail how energy is generated, transmitted and distributed, as well as areas of consumption, where such data are available. Conservation and management of energy are important for meeting specific targets and goals. Demand side management, energy conservation, energy efficiency and supply side management are common practices.

Supply-side management refers to the practice of electric utilities building generating plants to serve

whatever demand customers require. This practice was common until the 1970s and now includes incorporation of more efficient generation sources, either higher efficiency generators, or a diversified generation portfolio to include lower emission fuels, such as natural gas, or renewable energy, such as solar or wind power. Demand-side management is typically undertaken by a utility to change the level or timing of energy use on the customer side of the electricity meter, generally with the intention of optimising existing and planned generation and transmission assets on the utility side of the meter. It encompasses utility-initiated actions to influence consumer choices and behaviour and thereby improve energy efficiency, increase energy conservation, and reduce peak electricity demand. Rebates, incentives, and utility investments that improve building systems (such as the envelope, the efficiency of heating and cooling systems, or lighting systems) and rate structures that shift demand from times of peak energy use to off-peak hours can be deployed for demand side management activities.

Energy conservation is the process of using less energy. Energy conservation typically requires a change in behaviour (e.g. turning off lights when not in use, walking stairs rather than using an elevator, adjusting thermostats on air conditioners to use less energy). Behaviour change can be accomplished through education and awareness programmes. Implementing higher efficiency technologies or building components (e.g. cool roofing material) is another standardised practice for conserving energy across building portfolios. Energy efficiency is the process of producing more with the same or fewer units of energy through technological improvements. An example is replacing incandescent light bulbs with more efficient compact fluorescent lighting or light emitting diodes (LED), which provide the same amount of light but produce less heat, creating a more efficient light bulb. To understand the opportunities for efficiency and conservation, the grid system and energy consumption within Tonga are discussed in this section.

The 2017 installed capacity of the four island grid systems is shown in Table 1, by source and grid type. Tongatapu’s system represents the majority of the installed capacity, predominantly supplied by diesel fuel. As of June 2017, the total installed capacity on Tongatapu is 17.8MW, with 14 MW of conventional capacity from diesel generators, 2.3 MW from PV, 0.5 MW from wind, and a 1 MW battery storage system. In 2012, Tongatapu had 12.6 MW of conventional capacity, and 1.3 MW of renewables. The apparent growth in capacity is attributable to renewable-oriented policy ambitions, and increased electrical load, primarily driven by ownership of electrical appliances. Two additional renewable projects are currently under construction: a 1.3 MW wind farm funded by the Japanese International Cooperation Agency (JICA), and a 2 MW PV project developed by Chinese investors through power purchase agreements with TPL.

# Table 1: 2017 Installed generation sources (Source: MTED)

| Location | Grid Status | Conventional Capacity | RE Capacity | Battery Capacity (MW) |
| --- | --- | --- | --- | --- |
| Tongatapu | TPL Grid (11kV) | 14.00 MW | 2.8 MW | 1 MW |
| Vava’u | TPL Grid (Medium Voltage) | 1.87 MW | 0.42 MW | 0.2 MW |
| Ha’apai | TPL Grid (Medium Voltage) | 0.37 MW | 0.5 MW | 0.66 MW |
| Eua | TPL Grid (Medium Voltage) | 0.37 MW | 0.20 MW |  |
| Nomuka (Ha’apai) | Non TPL (6.6kV) | 0.09 MW |  |  |
| Ha’afeva (Ha’apai) | Non TPL (6.6kV) | 0.06 MW |  |  |
| Ha’ano (Ha’apai) | Non TPL (6.6kV) | 0.06 MW |  |  |
| ‘Uiha (Ha’apai) | Non TPL (6.6kV) | 0.09 MW |  |  |
| Household generators & solar home systems (Tongatapu) | Off-grid |  | 0.05 MW |  |
| Household generators & solar home systems (Ha’apai) | Off-grid |  | 0.06 MW |  |
| Household generators & solar home systems (Vava’u) | Off-grid |  | 0.13 MW |  |
| Household generators & solar home systems (Niuas) | Off-grid |  | 0.04 MW |  |
| TOTAL |  | 16.91 MW | 4.2 MW | 1.86 MW |

Figure 6 illustrates the gradual increase in capacity to meet rising demand. The figure also shows that solar output remains consistent month to month, while diesel generation decreases during the cooler months (June–August), and increases during the warmer months (December–February).

Figure 6: Energy generation mix for Tonga (monthly GWh)

(Source: TPL monthly generation data, June 2017)

Voltages vary across the four grid systems, as do also T&D losses. Voltages on Tongatapu are 11 kV for primary distribution; three phases: 415 V; single phase: 240 V. On Vava’u, Ha’apai, and ‘Eua, however, the voltages are 6.6 kV for primary distribution; three phases: 415 V; single phase: 240 V. Most of the primary distribution network on all islands consists of aerial lines supported by wooden poles, the exposure of which makes the grid vulnerable to both parasitic and line losses. Table 2 demonstrates that, although some improvements have been made on the outer islands, T&D losses remain relatively high.

# Table 2. Transmission and distribution losses

|  | TPL (Total) |  |  | Tongatapu |  |  | Vava’u |  |  | Ha’apai |  |  | ‘Eau |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2014 | 2015 | 2016 | 2014 | 2015 | 2016 | 2014 | 2015 | 2016 | 2014 | 2015 | 2016 | 2014 | 2015 | 2016 |  |
| Line Losses | 9% | 8% | 9% | 9% | 8% | 9% | 11% | 10% | 4% | 8% | 5% | 2% | 11% | 10% | 7% |  |
| Parasitic Losses | 2% | 3% | 3% | 3% | 3% | 3% | 2% | 2% | 2% | 2% | 2% | 2% | 2% | 2% | 2% |  |
| Billed | 89% | 89% | 88% | 89% | 89% | 88% | 87% | 88% | 94% | 90% | 93% | 96% | 88% | 89% | 91% |  |
| Source: TPL annual reports (2014/15, 15/16, 16/17) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The cost of generation varies from island to island due to diesel fuel transport costs. In 2009, however, TPL changed the structure across the four islands to create a more uniform price, providing the same retail tariff across all major islands. The cost per kWh on each island as of February 2018 was 0.8514 Tongan pa’anga (TOP). Figure 7 details the monthly retail electricity rate in Tonga, along with the associated cost of diesel fuel imported to Tonga where available. A spike in the cost of diesel in 2008 rapidly increased rates. In April 2017, in order to lessen the impact of rate increases on consumers, the government introduced a subsidised lifeline tariff, with a fixed rate at 0.7 TOP for the first 100 kWh of monthly consumption, applicable to all customers. While this rate structure may provide some economic benefit, especially to low-income households, it may also hinder the motivation for installing energy efficiency or renewable energy technologies and for practising energy conservation.

Figure 7: Electricity tariff and associated fuel costs

# (Source: TPL demand and prices, PICTS Electricity Tariff Database)

Energy is consumed by different sectors and the load profiles change according to the weather. TPL typically reads customer meters manually, taking up to 25 days to complete them, whereas other meters can be read remotely on the last day of every month. The difference in meter readings means that TPL uses a 12-month moving average to adjust for anomalies and determine total monthly generated power, parasitic losses and customer consumption. To improve reporting and power delivery, TPL is completing a meter replacement programme. According to TPL’s 2017 Annual Report, 3,000 single-phase meters and 500 three-phase meters were installed in 2016 as Phase 1 of a two-year project to replace customer meters to improve smart metering capabilities (i.e. remote, automatic meter reading). TPL is planning on upgrading all meters by 2018. The new meters are intended to provide alternative methods of billing and allow TPL to detect faults and power quality issues remotely. The smart metering programme may also help improve detection of parasitic losses or line losses. Line losses are determined by subtracting the power consumed by customer from power generated .

### Consumption

Tonga’s overall electricity  consumption is dictated by building design, building energy-using systems (such as appliances), and behaviour around electricity  consumption. In tropical areas such as Tonga, cooling loads in buildings has become critical, where the buildings are sealed and not open, especially in offices, supermarkets, banks, etc. The use of mechanical air-conditioning systems is widespread and increasing. Thermal comfort and indoor air quality (e.g. managing humidity and indoor pollutants) are more frequently being achieved by air-conditioning, rather than relying on vernacular architecture. The older, historic buildings in Nukuʻalofa were designed to utilise passive ventilation, have large overhangs to prevent solar gains from entering buildings and, as a result, use less energy than the more modern office buildings that rely on air-conditioning systems.

This section is broken into the four areas that are commonly associated with the highest electricity consumption: building design, residential end-uses, commercial end-uses, and street lighting.

#### Building design

Typically, electricity  consumption is dictated by the building design and by the equipment or appliances in use within each building type. For example, churches in Tonga are mostly open-air to allow for passive ventilation, rather than needing air-conditioning. This is a great building design to reduce the need for energy for cooling. Daytime services and activities also reduce the need for lighting. The largest energy end-use in churches is lighting, amplification systems (e.g. microphones), and electric fans in buildings that are designed to use natural ventilation and are open-air. TPL’s largest customer is a Church of Latter-day Saints complex built in 2007, which uses an average of 116,000 kWh per month, or 13.5% of the total billed consumption on Tongatapu. This level of consumption indicates an architectural design with closed windows and doors, and a dominance of air-conditioning loads. Similarly, office buildings that are not designed to use passive ventilation require air-conditioning to prevent moold growth, reduce humidity, and cool to a comfortable temperature. Commercial buildings, such as offices, use cooling and lighting technologies and also use computers, data processors, printers and fax machines, refrigerators, coffee pots, microwaves, etc. all of which generate heat internally, which increases the need to extract the heat for occupancy comfort or to cool the interior of the building.

Building codes were implemented in Tonga in 2007. The Building Control Division (BCD) of the Ministry of Infrastructure is responsible for planning and enforcement of policies and regulations in the building construction sector. BCD plans a complete revision of the current building code, its regulations and implementation/enforcement. It will increase building inspection activities and improve public awareness programmes to ensure that the general public and key stakeholders are updated on changes to the code and its regulations. The goal of the BCD is to “achieve more reliable, safe and affordable buildings and ensure structures are more inclusive, sustainable, affordable, safe and reliable.” One way the goals can be met is by updating building codes to specifically include energy efficiency and resilient buildings rather than vaguely referring to them as “safe and affordable”.

Energy efficient and resilient building technologies are deployed as a result of public policy. This is even the case when the cost of energy efficiency measures may not be significantly more expensive than traditional methods or when investments in energy efficient technologies result in energy savings. A study conducted by the U.S. National Institute of Building Science in 2005 determined that the return on the incremental investment in resilience pays off over the life of the building. The study concludes that for every dollar spent on hazard mitigation, an average of four dollars is saved during the disaster recovery phase. In December 2017, that report was revised to show that the savings are now a ratio of 1:6, rather than 1:4. Similarly, not only will updating building codes in Tonga save energy, it will also likely lead to fewer costs associated with disaster recovery in light of the anticipated severe weather (e.g. increased cyclones and severity of storm surges).  Climate change considerations are usually incorporated into building codes, along with energy efficiency requirements, and these  will also help with sustainable development and lower poverty levels associated with energy bills and recovering from severe climate-related events.

Some measures for  Tonga to consider are to create setback zone guidelines or a policy to protect buildings from sea-level rise and increasing storm surges, as demonstrated by tropical cyclone Harold on 9 April 2020. Elevated structures may be needed in certain areas to protect them from the same threats. Requiring the installation of attached features, such as roofing materials or solar PV panels, can also help protect against the higher wind levels associated with cyclones. Tonga may need to identify threats, vulnerabilities and risks; identify the technical measures to reduce the impacts associated with those risks; and integrate energy requirements at the same time.

Energy efficient and resilient building codes will address the following considerations:

- for new buildings, siting to reduce impact from climate-related threats (e.g. setback from shore, building height of no more than two stories);

- passive survivability techniques to reduce energy consumption and allow the building to be occupied without power (e.g. natural daylighting, natural ventilation, rainwater collection); and

- energy and water storage on site to allow for continued operation without utility services (e.g. solar PV with islanding controls and battery storage, cistern collectors with graywater systems).

- Design shape influences how wind moves around a building

- Roof pitch (6/12) can reduce lift and deflect wind

- Circular structure transfers wind pressure most efficiently

- Radial floor and roof trusses distribute weight and wind loading

- Wind is dispersed

- 2400 psi framing is twice as strong as typical framing material

- Five ply 5/8” sheathing instead of OSB exterior walls, roof and floor hardens buildings and prevents flying debris penetration

- Reinforced windows prevent wind and water infiltration

- Oversized truss hangers keep roof system anchored to walls

- Multiple construction ties add structural stability and transfer shear forces

- Continuous metal strapping increases structural stability

- Solar water heat provides on-site hot water

- Enhanced insulation maintains internal temperatures and reduces energy consumption; reflective coating or cool roof reflects heat gain

- Passive solar design helps cool building through appropriate window placement and roof overhang/shading

Rolling out codes in a way to first target critical infrastructure, such as schools, hospitals/medical clinics, evacuation centres, fire stations and police stations, will help communities be more resilient during natural disasters. Focusing on residential and commercial buildings after critical infrastructure will help to lower energy costs within those sectors, as well as make them more resilient. Codes should address new construction and renovations. Examples of codes can be found for the region and specific sectors, as well as studies on the topic of passive design codes for specific climates. An example of a cyclone resistant design with energy efficient and passive design is shown in Figure 8 for illustrative purposes.

Figure 8: Example of resilience measures for passive design and cyclone resistance (Source: Ecobuilding Pulse)

#### Residential end-use

The 2016 Population Census indicates there are 18,005 private households in Tonga, with Tongatapu having the largest number: 12,953 private households. According to TPL’s 2017 data, approximately 45% of the total load was from residential accounts. Residential accounts in tropical climates typically utilise energy for space cooling such as air-conditioning and fans; lighting; domestic appliances, such as televisions, clothes washing machines (labeled as Elec Washer) and electric water heaters; and kitchen appliances, such as fridges, freezers and cooking appliances. Census data indicate that the majority of Tongans cook with fuels rather than on electric burners, and also utilise a number of ancillary cooking devices (e.g. microwaves, electric kettles and food processers) which are captured by the Cooking category. Additionally, the majority of Tongans own energy efficient twin-tub washing machines, although census data indicate that, as GDP per capita increases, a higher percentage of Tongans are purchasing washing machines. In order to better understand how this electricity is being consumed, residential end-use consumption was modeled using the following methodology.

- The household ownership of various appliances was estimated using a variety of census data, surveys and proxy data.

- Data from TPL estimating average monthly consumption of appliances were calculated to estimate the total monthly consumption of households, based on the number of households that own a given appliance.

- These data were then normalised to TPL’s actual (and for future years, projected) total billed consumption, using the 2017 portion of total consumption from residential accounts.

- Using an estimate of appliance adoption rates tied to household GDP, residential consumption by end-use was modeled through 2030. The resulting data are displayed in Figure 9.

Figure 9: Residential electricity business-as-usual by end use

Due to Tonga’s geographic location, appliances are most commonly imported from Australia, New Zealand, Fiji, China and Singapore. The Minimum Energy Performance Standards (MEPS) or energy labeling schemes vary in each of these countries and the points of origin (i.e. places of manufacturing of the goods) may not be the same as the country from which the good was imported. This creates a challenge when determining the energy performance of appliances that are imported through Customs. Australia and New Zealand may have more stringent MEPS.

Figure 11: Various energy efficiency labels available in Tonga

1011111111111111

Figure 10: Origin of energy performance labels by level of recognition among Tongans (Source: Tebbutt Research)

Less expensive products with less stringent energy standards may, however, be available from Asia or via Fiji. In a 2017 Pacific Community (SPC) survey of Tongans, 58% had seen an Australian or New Zealand Energy Rating Label before, and the vast majority of these respondents said they had seen the label on an appliance in their own home, or in the home of a neighbour, or on a television advertisement. Figure 10 shows the percentage of respondents who recognised an energy performance label by source country when prompted with an example. An earlier study estimating the percentage of appliances in Tonga that already meet a MEPS is used as a baseline in modeling the effectiveness of new MEPS.

Although Tonga is currently in the process of implementing MEPS as coordinated by the Pacific Community under its Pacific Appliance Labelling and Standards (PALS) programme, exact performance requirements are unavailable. For this reason, modelling for the TEEMP uses standard consumption limits applied in Australia and New Zealand. In some areas, such as the curtailment of non-LED lights, the model extends beyond the MEPS imposed by Australia and New Zealand. Figure 11 shows the various labels included on appliances sold in Tonga.  Table 3 uses customs data to filter appliances imported between 2009 and 2017 by country of origin. This is likely a good estimation of appliances that already meet another country’s MEPS.

Table 3: Appliances imported by country of origin (Source: Tebbutt Research)

| Item | Country of Origin | Number Imported (2009-2017) |
| --- | --- | --- |
| Air Conditioner | New Zealand / Australia | 325 |
|  | Singapore | 15 |
|  | China / Hong Kong / Taiwan | 985 |
|  | Other | 2025 |
| Fan | New Zealand / Australia | 6137 |
|  | Singapore | 510 |
|  | China / Hong Kong / Taiwan | 8455 |
|  | Other | 10941 |
| Freezer | New Zealand / Australia | 3423 |
|  | Singapore | 825 |
|  | China / Hong Kong / Taiwan | 4641 |
|  | Other | 3172 |
| Fridge | New Zealand / Australia | 3281 |
|  | Singapore | 853 |
|  | China / Hong Kong / Taiwan | 433 |
|  | Other | 1594 |
| Stove or Oven | New Zealand / Australia | 209 |
|  | Singapore | 116 |
|  | China / Hong Kong / Taiwan | 673 |
|  | Other | 990 |
| Solar Components | New Zealand / Australia | 1555 |
|  | Singapore | 57 |
|  | China / HK / Taiwan | 5132 |
|  | Other | 8447 |
| Water Heater | New Zealand / Australia | 325 |
|  | Singapore | 16 |
|  | China / HK / Taiwan | 206 |
|  | Other | 256 |

In an assessment funded by Australian Aid, scenarios were formulated around the business-as-usual (BAU) activities in Tonga and implementing minimum performance standards for lights and appliances. Scenarios were used to compare the energy savings in GWh consumed through 2030. In Australian Aid’s study, implementing MEPS early on will achieve a 10 GWh savings by 2030. The TEEMP analysis anticipates that upwards of 12.7 GWh could be saved through rigorous MEPS implementation by 2030; the difference between this figure and Australian Aid’s figure can be attributed to different BAU consumption figures, less selective MEPS, and applicability to fewer appliances.

This analysis, coupled with information on the next major appliances expected to be purchased from the SPC survey, can lead to further anticipation of MEPS savings. Clothes washers, televisions, refrigerators and freezers make up most appliances purchased by Tongans and also the future  anticipated appliance purchases. Focusing on MEPs for those areas, as well as air conditioners, can lead to energy savings overall. Figure 12 gives an indication of the current appliances installed and anticipated appliances to be purchased.

# Figure 12: Tongans reported last major appliance purchase (left) and next anticipated appliance purchase (right)

In an assessment funded by Australian Aid, scenarios were formulated around the business as usual (BAU) activities in Tonga and implementing minimum performance standards for lights and appliances. Scenarios were used to compare the energy savings in GWh consumed through 2030. In Australian Aid’s study, implementing MEPS early on will achieve a 10GWh savings by 2030. The TEEMP analysis anticipates upwards of 12.7 GWh could be saved through rigorous MEPS implementation by 2030; the difference between this figure and Australian Aid’s can be attributed to different BAU consumption figures, less selective MEPS, and applicability to fewer appliances.

Growth in appliance uptake is largely tied to household GDP per capita. A previous study looking specifically at Asian-Pacific countries identified an approximate 3% growth in the ownership of air conditioners for every USD 1,000 increase in household GDP. In the modelling, this growth percentage was applied equivocally to all appliance uptake, with caps placed on appliance ownership based on ownership in American Samoa, a country with plentiful data availability and a comparable GDP per capita to that projected for Tonga in 2030.

Goods were observed to be labeled in retail spaces in October 2017 with labels indicating the amount of energy that might be consumed, which is a good practice. Expanding labelling across all appliances, and even potentially lightbulbs, could have upwards of an 11% reduction of energy consumption in Tonga, according to one study. The effectiveness of MEPS and labelling is, however, highly dependent on spreading public awareness of how to read energy efficiency labels and understanding the importance of conserving electricity.

Data were synthesised from the preceding sources, including the census, consumer research, and customs data, to estimate current residential appliance ownership rates and to forecast future adoption. Figure 13 shows that some appliances with an already high ownership percentage, such as fridges, freezers, fans and televisions, will see modest growth between 2018 and 2030. More electric intensive domestic appliances, such as air conditioners, electric clothes washers, and electric water heaters, were projected to see larger growth over the same period. Although these appliances do not currently represent a significant portion of residential load, foresight in implementing MEPS on these appliances ensures that non-efficient appliances are not imported for use over a 15–20-year appliance life cycle.

Figure 13: Residential appliance ownership by year

#### Commercial end-use

While Tongan data were available to estimate the percentage of load by end-use for residential accounts, similar data were not available for commercial accounts. To fill this gap, an energy model was run for a similar climate zone to understand energy end-use within a typical commercial building. Using data from an energy audit conducted in Marshall Islands, an energy model was created to represent a sample office building in Tonga to serve as an energy baseline for commercial buildings. An energy baseline is essential to understand what the current energy consumption is, where the highest consumers are (e.g. HVAC, lights, fans, pumping, etc.) and to calculate potential energy savings associated with identified energy efficiency opportunities. Using the eQUEST building energy model, the existing operating conditions of air-conditioning, lighting and plug load systems were modeled. A graphical representation of the building energy model developed in eQUEST is shown in Figure 14. The geometry of the building was simplified for modeling purposes, while still allowing accurate simulations of energy transfer through all surfaces in the building.

Figure 14: eQUEST model representation
(Source: Photo Jimmy Salasovich, NREL; Energy model image generated using eQUEST)

Estimates of the current energy use breakdown at the sample office building in Tonga were generated, using the eQUEST building energy model, paired with weather files to simulate realistic climate conditions, shown in Figure 15. As expected, space cooling (blue) is the largest electricity consumer, followed by lighting (yellow) and ventilation fans (magenta) for a sealed, air-conditioned building. Plug loads (e.g. office equipment) are shown in green and are a smaller component of the building’s energy consumption.

Figure 15: eQUEST baseline energy use estimates
(Source: Figures generated using eQUEST)

These data were applied uniformly to the commercial portion of TPL’s projected commercial load growth to estimate the yearly appliance consumption by end-use. This is shown in Figure 16. TPL’s highest consuming customers are churches, government buildings, telecom companies, public services, bakeries, hotels, universities and hospitals. The methodology used in the model applies the end-use data from a single building across all of these accounts. While other methodologies could be used to more accurately model commercial end-use, data are unavailable to estimate what portion of the commercial load is going to different account types (government, industrial, retail, etc.) or to estimate yearly appliance uptake of commercial accounts. It is recommended  for TPL to begin recording data of this sort in order to better understand its commercial load and enable it to conduct energy audits.

Figure 16: Commercial electricity business-as-usual by end-use

#### Street lighting

The last major sector of electricity use in Tonga is street lighting. Tonga does not use traffic lights, opting for energy efficient roundabouts. The street lighting installed is typically near town or village centres and along major junctions or intersections. There is a programme to convert older, less efficient street lights to higher efficiency LED lamps to save energy. According to various reports, Tonga coverted the following number of street lights to LED each year:

    - Tongatapu: 135 existing street lights replaced with high efficiency LED units (2015);

    - Tongatapu, Vava’u, ‘Eua, Ha’apai: over 1,500 LED lights installed (2016);

    - Tongatapu, Vava’u, ‘Eua, Ha’apai: over 1,300 LED street lights installed (2017); and

    - outer islands: all 161 existing street lights replaced with high efficiency LED units.

Figure 17: Example of LED street light

Besides energy savings associated with replacing higher energy use lights with more efficient lights, an added benefit associated with these projects is more accurate colour rendering. As seen in a streetlighting upgrade project in Figure 17, the light cast from high pressure sodium is yellow and makes it difficult to see the true colours of objects. The after LED picture shows more accurate colours, which is beneficial not only for better night-time vision, but also increases safety and reduces crime. One additional benefit of LED lights is a longer lifespan, which also reduces operations and maintence costs associated with replacing bulbs.

Understanding energy generation, transmission and consumption will assist with identifying opportunities for reductions and efficiencies in the wedge analyses. This analysis is covered in further sections.

## Baseline transportation

The analysis in this section focuses on land transportation because that is the most clearly identified priority for Tonga, it is the most under Tongan control, and it is growing at a much faster rate than aviation and maritime loads (as shown in Appendix A). To properly baseline the transportation fuel use in Tonga, it is important to first document the fuel used and trends thereof. Then it is possible to estimate the annual miles travelled and assess what vehicles were doing the travelling.

The baseline for Tonga’s transportation activities and fuel consumption are summarised in this section, organised by fuel usage, kilometers travelled, and other assessments.

### Land transportation fuel usage

Fuel usage has been tracked and compiled by the MTED. Starting in 2012, MTED divided petroleum usage into petrol, diesel, Jet A1, and kerosene, and divided end uses into service stations, aviation, commercial, government and power stations. It is assumed that all diesel and petrol being used in the service stations, government and commercial sectors is land transportation fuel, and the summarised consumption in million litres is shown in Figure 18.

Figure 18: Land transportation fuel consumption in Tonga (

# Source: MTED fuel volume 2012–2015 report)

### Vehicle kilometers travelled

The fuel reported in the above section is used to propel motorised vehicles. The distance they are propelled is measured in vehicle kilometres travelled (VKT). VKT reflects simultaneously on how efficient vehicles are (getting more VKT per litre of fuel) and how dependent a population is upon motorised vehicles (extremely dependent populations have a high per capita VKT).

Tonga’s annual VKT was derived from the traffic count map of Tongatapu taken in 2016 by the Ministry of Infrastructure. The counts are admittedly incomplete, but they are the best data available to estimate VKT. The process, the assumptions needed, and the data checks are outlined in this section and a map of the traffic count is shown in Figure 19.

Figure 19: Traffic count map of Tongatapu (

# Source: Ministry of Infrastructure)

To convert the traffic counts into VKT, traffic was tracked as it was leaving Nuku’alofa. This reflects the assumption that most traffic is going and coming from Nuku’alofa (with exceptions explained later). The methodology assumes that all vehicles are departing Nuku’alofa, which is the same as acknowledging that most vehicles are counted twice daily and have the same distance on both trips (generally the morning commute and the evening commute). Table 4 shows the traffic going through given counters as it departs Nuku’alofa.

The first three columns are the same as shown on the map legend. The next column reflects the assumed primary destination (Destination A) of the vehicles going through the counter. Destination A was often a populated town, region, or the next counter. Destination B was typically a secondary population cluster beyond Destination A. The assumed number of vehicles was then multiplied by the kilometres to the destination to give the daily VKTs going to each destination. These VKTs are listed in the two Daily VKT columns that are then aggregated to the grand total daily VKT. The last two columns estimate the rural trips that did not go to, from, or through greater Nuku’alofa or along Taufa’ahau Road. This traffic was primarily within the Hihifo and Hahake districts. Estimates were developed with local consultation and the end VKT accounts for 12% of all miles on Tongatapu. These numbers are the most subjective but have a limited impact on the overall estimate.

Table 4: Traffic counts, traffic destinations and daily VKT on Tongatapu

| Road | Counter Location | VEH's/day | Destination A | Kilometres to A | Vehicles to A | Daily VKT | Destination B | Km's to B | VEH's to B | Daily VKT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Taufa'ahau Road | Fanga | 21,528 | Tofoa | 1.2 | 15,837 | 19,111 | Havelu & Vaoloa | 0.5 | 5,691 | 3,022 |
| Bypass Road | MOI | 9,822 | Western suburbs | 2.5 | 4,911 | 12,169 | Hihifo district | 10.0 | 4,911 | 48,991 |
| Fatafehi Road | OE | 7,319 | Intra-Nuku traffic | 2.5 | 7,319 | 18,135 | - | - | - | - |
| Taufa'ahau Road | Pasilika | 10,866 | Intra-Nuku traffic | 2.5 | 10,866 | 26,924 | - | - | - | - |
| Vaha'akolo Road | Queen State College | 7,175 | Sopu | 2.5 | 7,175 | 17,779 | - | - | - | - |
| Matealona road | THS | 10,830 | Western suburbs | 1.2 | 5,415 | 6,535 | Hihifo District | 10.0 | 5,415 | 54,019 |
| Tupoulahi | Ngele'ia | 7,162 | Intra-Nuku traffic | 2.5 | 7,162 | 17,746 | - | - | - | - |
| Vuna | Chinese Embassy | 15,600 | Ma'ufanga and peninsula | 2.5 | 15,600 | 38,655 | - | - | - | - |
| Fatafehi Road | TDB | 5,558 | Intra-Nuku traffic | 2.5 | 5,558 | 13,772 | - | - | - | - |
| Wellington | Perma&Sons | 5,381 | Intra-Nuku traffic | 2.5 | 5,381 | 13,333 | - | - | - | - |
| Tuku'aho | Pelehake | 2,781 | Fua'amotu, Lavengatonga, and coast towns in between | 5.3 | 2,781 | 14,766 | - | - | - | - |
| Taufa'ahau Road | Malapo | 4,039 | Tatakamotonga | 4.8 | 2,020 | 9,748 | NE coast towns | 13.0 | 2,020 | 26,320 |
| Vaini-Longoteme Rd | Vaini | 1,252 | Accounted for in Pea line | - | - | - | - | - | - | - |
| Taufa'ahau Road | Vaini Police Stn | 9,298 | Accounted for in Pea line | - | - | - | - | - | - | - |
| Vaini-Longoteme Rd | Sei'uhila | 554 | Accounted for in Pea line | - | - | - | - | - | - | - |
| Folaha Road | Nualei | 1,012 | Accounted for in Pea line | - | - | - | - | - | - | - |
| Taufa'ahau Road | Pea | 12,400 | Viani | 7.2 | 10,550 | 76,387 | Folaha* | 10.5 | 1,566 | 16,378 |
| Fonongahina | Pea | 4,451 | West on Taufa'ahau | 6.4 | 4,451 | 28,647 | - | - | - | - |
| Vaha'akolo | Tofoa | 7,988 | Pea | 1.6 | 4,451 | 7,162 | - | 2.5 | 3,537 | 8,821 |
| Taufa'ahau | Tofoa | 13,201 | Pea | 1.6 | 12,400 | 19,952 | - | 2.5 | 801 | 1,998 |
| Intra-Hihifo trips | No Counters** | - | - | 4.8 | 5,000 | 24,135 | - | - | - | - |
| Intra-Hahake trips | No Counters** | - | - | 4.0 | 5,000 | 20,113 | - | - | - | - |
| Total Daily VKT on Tongatapu |  | 545,531 |  |  |  |  |  |  |  |  |
| *284 of these cars are assumed to continue 3.2 km past Folaha on to Vietongo and Coast ** There were no counters in the Hihifo or Hahake districts. See text for how the vehicle numbers were estimated. |  |  |  |  |  |  |  |  |  |  |

The results of Table 4 are that Tongatapu sees 545,531 VMK every day. Assuming that this flow is consistent six days per week (Monday through Saturday, excluding Sunday for the religious holiday), this aggregates to nearly 171 million VKT every year. In order to extrapolate from Tongatapu to all of Tonga, a link was made between VKT to fuel use. Fifteen per cent of land transportation fuel is used on islands other than Tongatapu, so it was assumed that 15% of VKT did also. Taking this into account results in the estimate of 2,289 VKT per capita – a realistic number, given its per capita gross domestic product (GDP) and other parameters. Furthermore, when dividing the fuel used in Tonga by this VKT, the result is an average vehicle fuel consumption rate of 12.5 L/100 km – a figure that is quite realistic for the vehicle population.

### Vehicles

The third key piece of information for defining the land transportation picture of Tonga is the number and type of vehicles. Tonga has four sources of vehicle information, each with certain strengths and weaknesses.

- Census data of household goods (from the 2011 and 2016 Census). This has the clear advantage of showing vehicle stock instead of incremental vehicles. However, it is focused on households and therefore misses many of the corporate, government and religious institution vehicles. Furthermore, its vehicle types are not as refined as the vehicle registrations (2011 has only two categories for motor vehicles while 2016 breaks them down into six categories).

- New vehicle registrations (2010–2016), as tracked by the Ministry of Infrastructure and reported by the National Reserve Bank of Tonga. This source has the advantage of good categorisation, which enables us to better estimate the impact of policies and projects that target specific vehicle categories. However, it reports only new vehicle registrations, while vehicle stock is needed.

- Vehicle imports tracked by Tonga Customs. These data are not divided into vehicle category and are therefore not as useful for fuel use estimates. However, they do provide a good data check on the registrations.

- Government fleet, tracked by the Ministry of Infrastructure. These data show 538 vehicles in the government fleet, broken down by vehicle type in Figure 20.

Figure 20: Government vehicle inventory by type

(

# Source: Ministry of Infrastructure 2016/17 Government Vehicle Inventory)

The following steps were employed to utilise the available data to estimate the rolling stock of vehicles in Tonga.

- Focus was placed on vehicle registrations, enhanced with the government fleet data, because both of these datasets were broken down by vehicle type and they were largely in agreement with vehicle imports.

- In order to convert new registrations into vehicle stock, the length of time a vehicle has been in Tonga and is used before retirement was taken into account. For this, Census 2016 data were used, showing that 59% of households owned vehicles and 8% had purchased them within the past year. This would indicate that the average vehicle lasts 59/8=7.3 years after being imported.

- The 2010 data were removed because it was an outlier year of heavy vehicle replacement after Cyclone Rene. This outlier would have inaccurately affected the vehicle stock calculations because many of the additional vehicle registrations were replacing destroyed vehicles.

- Vehicles that pre-date the dataset are still part of the current stock, so an estimate was calculated as to what registrations data were available before the dataset. Therefore, the projected sales patterns were calculated backwards from 2011 to 2003, based on later year registrations and money spent on vehicle imports as tracked by Tonga Customs.

- The “government vehicles” category was replaced by more granular data on government vehicles, and these vehicles were allocated to their weight-based categories.

Table 5: Tally of vehicles in Tonga by type

| Vehicle Type | Number of Vehicles |
| --- | --- |
| Cars | 6,031 |
| Light trucks, vans, SUVs | 7,103 |
| Heavy duty vehicles | 2,099 |
| Taxis and rentals | 953 |
| Motorcycles | 306 |
| Buses | 225 |
| Total | 16,717 |

- Comparisons were made between the new dataset, the imports data, and the 2016 census data. The total number of vehicles is 3.5% less than if the calculations had been based on the annual vehicle imports. Conversely, the total of vehicles is 13% greater than the combination of 2016 Census data and the government fleet data. It is expected that the actual number of vehicles would be greater than that derived from the census data, since many of the vehicles are owned by corporations (which were not included in the census) and not households.

### Other baseline data of interest

While the 2006 and 2011 censuses asked Tongans about the ‘availability’ of mobile telephones in households, the 2016 census asked about household ‘ownership’ of cell phones. The difference in this methodology accounts for a reported decrease in the number of cell phones, from 73% in 2006, to 94% in 2011, down to 89% in 2016. It is unclear what proportion of these phones are ‘smart phones’, capable of accessing the internet for transportation purposes.

The 2016 Tonga census asked respondents about the number of bicycles owned. It reported that 4,695 households (26%) own a bike, and that 7,009 bikes are owned overall. Previous censuses did not ask respondents about ownership of bicycles.

The Ministry of Infrastructure has provided yearly data on the number of domestic ferries and their passengers, along with the volume of cargo delivered to various wharfs. These data will be incorporated to examine how additional ferry and domestic sea travel could affect Tonga’s fuel use and carbon emissions.

Tonga residents reported 695 motorcycles in the 2011 census, yet that number dropped to 275 motorcycles in the 2016 census – a drop of 38%. The reasons people gave up this efficient mode of transportation should be understood as part of a multi-faceted approach to improve transportation fuel efficiency.

The ideal data collection system would consist of an annual vehicle registration that includes odometer readings of vehicles. This would provide two key pieces of information that were derived above: vehicle stock and annual VKT.

Business-as-usual projections

In order to identify areas of reduction and effectiveness of various conservation scenarios it is important to create business-as-usual (BAU) projections. Where data were available, the information was used to create historic trend lines to help with projections. Where no data were available, either for the energy sector or the transportation sector, assumptions were made and have been documented in this section.

## Tonga transportation fuel business-as-usual projection

Historically, the period of greatest growth in VKT, and therefore land transportation fuel use, is when a country’s per capita GDP (in international dollars) is between USD 5,000 and USD 20,000. In other words, this is the range where VKT growth is on the steep part of the S-curve (see Figure 21 below). Tonga’s per capita GDP crossed into that zone in 2015 (see Figure 22). Therefore, Tonga’s VKT should now be very sensitive to increases in per capita GDP. Furthermore, since Tonga has shifted into this rapid growth phase, it is better to base projections on what other countries have done throughout this phase, rather than Tonga’s previous growth rates.

Figure 21: VKT of motorised passenger travel and per capita GDP for 22 developed countries

(Source: Rand 2014)

(Note: These numbers omit freight and focus on passenger travel. Geary-Khamis dollars are the same as the international dollars described earlier in this report)

Since land transportation fuel use is most closely tied to VKT, and VKT is most closely tied to per capita GDP in international dollars, a projection of Tonga’s per capita GDP is needed into the future. The per capita GDP data from 1980 to 2017 are shown in Figure 22 (in blue). The line of best fit is linear (with R2=0.979) and therefore future projections are based on that line. This line shows Tonga’s per capita GDP reaching $9,000 in 2050.

Figure 22: Tonga per capita GDP with purchasing power parity

# (Data source: Knoema Tonga GDP per capita)

Next, it is essential to convert the projected per capita GDP into VKT. To do that, it is crucial to translate the starting point of 2,289 VKT per capita (including freight) into a number that excludes freight and only includes passenger travel. In New Zealand, approximately 23% of the total VKT are freight, so it is assumed that this ratio is the same for Tonga. Subtracting 23% from the total VKT of Tonga leaves 1,750 VKT of motorised passenger travel per year.

In order to project the starting point of 1,750 VKT/year to 2050, the correlation between Tonga and other countries developing and growing their per capita GDP from $5,000 to $9,000 is considered (as shown in Figure 22). Countries represented on the chart showed different relationships between GDP and VKT, and these differences were largely due to spatial dispersion, car infrastructure, fuel prices, and alternatives to driving. Using a simplified version of the process described in RAND 2014, it was determined that, in 2050, assuming that Tonga will have a per capita GDP of $9,000, it will have a VKT of 3,900 per person. This number appears slightly high when comparing it to Figure 21 for two reasons. First, Tonga GDP is in 2017 $Intl instead of 1990 $Intl as the rest of the figure. Second, the Tonga VKT appears high when compared to the other countries charted in Figure 21 because Tonga is starting its VKT growth spurt decades later. Therefore, developments in vehicles, infrastructure, and the shipping of materials and fuel have positioned Tonga for more rapid growth than the countries that developed in the 1960s and 1970s. This relative advantage is validated by the fact that Tonga VKT is already well above where other countries were when they had a GDP of $5,000. Using the same assumption that 24% of a country’s VKT is due to freight, that factor was added back in to bring Tonga’s total 2050 VKT to 5,103 per person.

Growing VKT from 2,289 VKT/person in 2016 to 5,103 in 2050 requires a compounded annual growth rate of 2.4% per year. This growth rate was applied to the 2015 fuel consumption quantities, which resulted in the fuel projections shown in Figure 21. In order to do so, the following assumptions were made.

- Tonga’s population remains constant between 2017 and 2050. This is a reasonable assumption given that Tonga’s average annual growth between 2011 and 2016 was ₋0.5%.

- Tonga’s vehicle efficiency remains the same as it is now, with an average of 12.5 L/100 km. This is a standard assumption when developing a business-as-usual scenario.

- Tonga’s fuel use ratio remains the same as it is now, with 57% petrol and 43% diesel.

- Tonga’s growth in VKT, and its growth in fuel use, grows at 2.4% per year, compounded annually.

Figure 23 : Tonga BAU transportation fuel usage

Fuel use was then converted to lifecycle GHG emissions, based on conversion factors in EPA 2016 and EIA 2016. The end result of this projection, with associated assumptions, can be seen in Figure 24. It is important to note that the BAU, in terms of diesel, is based on 500 ppm diesel, while the current diesel used in land transport has been 10 ppm since 2018.

Figure 24: Tonga BAU land transportation GHG emissions

## Tonga electricity business-as-usual projection

Data from the TERM were compared to data received from TPL’s annual reports in order to determine the best projection scenarios for all of Tonga, recognising that the largest growth is predicted to occur on Tongatapu. The energy billed and peak demand for each of Tonga’s four island groups is shown in Table 6. As more remote islands are electrified, populations may shift on those islands, as young adults may choose to move closer to family or new businesses may start in those remote island areas (e.g. tourism or resorts with access to power). Not knowing what the future holds for each of the islands, a general projection was made, based on total population growth. Historical data shows fluctuations in billed energy at the customer level with a ₋6.72% change from June 2009 to June 2010 and a 7.53% increase from June 2015 to June 2016, as shown in Table 7. Projections in the TERM indicate an estimate of projected growth by 2020 of 28% from 2010 consumption levels.

Table 6: Billed energy (MWh) and peak demand (MW) in Tonga’s four island groups (Source: TERM)

|  | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 |
| --- | --- | --- | --- | --- | --- | --- |
| Tongatapu grid |  |  |  |  |  |  |
| Billed energy, MWh | 36,759.00 | 36,223.30 | 36,001.00 | 36,760.10 | 38,230.60 | 40,142.10 |
| Peak demand, MW | 7.903 | 7.654 | 7.5 | 7.565 | 7.815 | 8.152 |
| Vava'u grid |  |  |  |  |  |  |
| Billed energy, MWh | 4,037.00 | 3,978.10 | 3,953.70 | 4,037.10 | 4,198.60 | 4,408.50 |
| Peak demand, MW | 1.021 | 0.989 | 0.97 | 0.981 | 1.013 | 1.058 |
| Ha'apia grid |  |  |  |  |  |  |
| Billed energy, MWh | 1,236.50 | 1,240.00 | 1,255.40 | 1,294.20 | 1,346.00 | 1,413.30 |
| Peak demand, MW | 0.3 | 0.3 | 0.304 | 0.313 | 0.325 | 0.341 |
| 'Eua grid |  |  |  |  |  |  |
| Billed energy, MWh | 882.1 | 884.6 | 895.6 | 923.3 | 960.2 | 1,008.20 |
| Peak demand, MW | 0.229 | 0.227 | 0.225 | 0.228 | 0.235 | 0.245 |
| Total, all grids |  |  |  |  |  |  |
| Billed energy, MWh | 42,914.50 | 42,326.00 | 42,105.60 | 43,014.70 | 44,735.30 | 46,972.10 |

Source: TERM

Table 7: Historical TPL reports on energy billed and consumed by customer (2008–2017)

|  | July 008–June 2009 | July 2009– June 2010 | July 2010– June 2011 | July 2011– June 2012 | July 2012– June 2013 | July 2013– June 2014 | July 2014– June 2015 | July 2015– June 2016 | July 2016– June 2017 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Total billed (MWh) | 44,539 | 42,625 | 44,566 | 44,731 | 46,388 | 49,165 | 49,165 | 53,342 | 58,823 |
| Total generated (MWh) | 54,659 | 51,845 | 53,160 | 52,391 | 53,313 | 55,405 | 55,405 | 60,037 | 66,519 |
| Customers | 20,338 | 20,773 | 20,758 | 20,498 | 20,580 | 20,633 | 20,932 | 20,999 | 22,929 |
| MWh billed per customer | 2.19 | 2.05 | 2.15 | 2.18 | 2.25 | 2.38 | 2.35 | 2.54 | 2.57 |
| Percentage change |  | ₋6.72% | 4.42% | 1.6% | 3.18% | 5.4% | ₋1.45% | 7.53% | 0.98% |

TPL provided their internal projection of load growth, which is replicated below in Figure 25. Ultimately, these data were chosen to represent the total yearly generation in the BAU because of TPL’s first-hand experience operating Tonga’s grid. When compared with the preceding tables, TPL’s projection appears to match a moving average of historical growth, making it inclusive of factors such as population growth. Because of varied population growth estimates, this was not included in the model as a separate variable to model load growth. Household appliance ownership data, and commercial load models were then used to estimate yearly consumption by end-use as described in Section 2.1. Using a similar methodology to the transportation BAU, the electricity BAU was brought to the common metric of CO2e emissions, which is shown in Figure 26.

Figure 25: TPL projected load growth (Source: TPL)

Figure 26: Projected electricity emissions under BAU and with policy changes

Proposed reduction targets

Tonga’s INDC calls for 50% renewable penetration in the electricity generation sector by 2020 and 70% by 2030 but does not take into consideration  GHG emissions from transportation or efficiency improvements. This section aims to incorporate these two sectors and propose new GHG reduction targets. It will do so by dividing the targets into smaller sub-components and presenting them in a way that  Tonga can deem as realistic or unrealistic and adjust accordingly. This part is known as a wedge analysis, due to the visual aids that track the difference between BAU scenarios and the path to target achievement. It will then discuss policy and project options that enable achievement of these targets.

## Transportation wedge analysis

GHG emissions from Tonga land transportation can be reduced through VKT reduction, vehicle efficiency improvement, idle time reduction, and alternative fuels, such as electricity or biodiesel. This section introduces each of these methods, proposes related goals, and assesses their contribution through a wedge analysis. These related goals are a middle step between the overall GHG reduction target and the projects and policies to achieve that goal. As such, these sub-goals are a tangible way to check the feasibility of an overall GHG reduction target. Each sub-goal, as reflected in Figure 27, aggregates to achieve the overall emissions reduction target. It should be noted that improving vehicle efficiency through traffic flow improvement is another method for reducing GHG emissions, but this has not been included because at Tonga’s stage of development there is a large and uncertain takeback effect where traffic flow improvements lead to increased VKT.

Figure 27: Transportation wedge analysis, thousand metric tonnes of CO2e

Figure 27 shows that, if each of the sub-goals is achieved, it would lead to a 28% reduction in GHGs below BAU by 2030. This is actually a 1% increase above the 2018 baseline. The proposed sub-goals, listed below, all refer to reduction from the 2030 BAU case.

- 20% reduction in VKT of light-duty vehicles (LDVs) through walking, biking, transit, rideshare, telecommute. This sub-goal focuses on LDVs because these vehicles are responsible for the majority of emissions increases as a country develops. It should be noted that a 20% reduction from the 2030 BAU VMT is equivalent to a 15% reduction from the 2018 VMT.

- 30% improvement in fuel economy for new LDVs through registration fees, import tariffs, or fuel economy standards. This would be an improvement from 10.1 L/100 km in 2016 to 7 L/100 km in 2030. This sub-goal is realistic because it is slightly less ambitious than what the U.S. achieved through its Corporate Average Fuel Economy programme between 2010 and 2017 (34% improvement in 7 years, starting at 10 L/100 km).

- HDV idle time reduced by one hour per day. HDVs use about 3.5 litres of diesel per hour and LDVs about 1.5. This high consumption rate is due to the fact that engines are not operating within their optimal range when idling. Auxiliary loads such as air-conditioning or hydraulic lifts require only a fraction of the power that is delivered from an idling vehicle engine. Many cities around the world have successfully implemented idle bans that have reduced HDV idle time by much more than one hour per day. Tonga has an advantage, since its warm temperatures enable easy diesel start-up without concerns for gelling fuel. Numerous technologies can enable significant idle time reduction, including automatic engine shut-off, storage cooling, axillary power units, electrified parking spaces, and batteries.

- All road transportation diesel contains 10% biodiesel. This amount would be sourced from all available waste oil and grease in the country (which could provide an estimated 77% of the targeted biodiesel in 2030) and the rest would come from coconut oil. The Philippines are quickly approaching 5% coconut biodiesel in their large diesel market and have advanced the science and production methods along the way. In this example, coconuts were the preferred  source of biodiesel because they are symbiotic with many other crops.

- Ten per cent of new LDVs are electric by 2030. This is a very conservative goal, for two reasons. First, batteries are the dominant cost of EVs, and these costs are dropping quickly. Secondly, the efficacy of EVs as a GHG-mitigating technology largely depends on the electricity mix. Figure 28 compares the carbon emissions of EVs, conventional vehicles, and a Prius HEV. The Nissan Versa and Nissan Leaf are the conventional and EV versions of the same model. Likewise, the Toyota Highlander and Tesla Model X are the conventional and EV versions of SUVs of similar size. Both EVs are lower-emitting than their conventional counterparts, even with 0% renewable electricity. However, this chart also highlights the fact that HEVs, such as the Toyota Prius, have low emissions, regardless of the electricity mix. Therefore, the EV-promoting policies and projects highlighted in Table 8, such as the purchase rebates and the mechanic training, should be inclusive of HEVs, PHEVs, and EVs and the market can determine which is favoured at any given time.

Figure 28: GHG Emissions benefits of electric vehicles in Tonga

## Electricity wedge analysis

Tonga’s INDC includes a commitment to replace 50% of electricity with renewables by 2020 and 70% by 2030; Tonga’s Strategic Development Framework (published the same year as the INDC, 2015) states the goal as 50% by 2025. While the  goals are ambitious, indications are that they will be met. To achieve the 2020 goal, a significant number of renewable energy developments are currently under way and more are planned, including the installation of battery storage systems. To account for some potential slippage, the electricity model uses the 50% renewable by 2025 target, and then maintains that level of renewable penetration through 2030. Efforts to reach 50% renewable before 2025, or to exceed 50% renewable henceforth should be applauded, but using the 2025 target provides a more achievable target for the model.

Tonga’s ambitious renewable plan will already significantly reduce the country’s emissions from electricity. Pairing this plan with a suite of energy efficiency policy options offers the potential to not only further reduce Tonga’s emissions, but to also reduce the amount of new renewable capacity needed to meet penetration targets. To achieve 50% renewables by 2025 without any MEPS or policy options, Tonga would need the equivalent generation from approximately 120,213 solar panels; when the suggested energy efficiency policy options and MEPS are implemented, that number drops to 79,260 panels, a capacity reduction of 34%. In this model, a 70% reduction in Tonga’s 2030 electricity emissions – from 91.5 thousand metric tonnes CO2e to 31.4 thousand metric tonnes CO2e – is possible. This signifies a reduction of 60.1 thousand metric tonnes CO2e, of which 49% is from increased renewable energy penetration, 16% is from stringent MEPS reductions, and 35% is from a suite of other policy options.

Figure 29: Electricity wedge analysis

In order to meet this target, a number of sub-targets should be established. These are listed below.

- One hundred per cent adoption of MEPS by 2022 (based on a seven-year phase-in of policies); extend MEPS to AU/NZ standards for all fridge/freezers, ACs, water heaters, TVs, computers, clothes washing machines/dryers, cooking appliances, and curtail importation of non-LED bulbs. Implement 100% labelling of appliance efficiency.

- Fifty per cent renewable energy penetration by 2025. Incentivise adoption of TPL’s existing net-metering programme, especially for high-use customers. By generating electricity on site, additional benefit can be realised by avoiding line losses.

- Reduce line losses to the lowest feasible amount by 2030. Great strides have already been made at this in recent years, and the introduction of smart meters should help identify the largest problem areas.

- Implement new EE and cyclone resistant building standards and perform energy audits on all major energy consuming customers by 2025.

- Reduce the uptake of electric hot-water heaters, particularly those with tanks, by incentivising the purchase of solar hot-water heaters.

- Replace 100% of streetlights with LED bulbs, and use LED bulbs in government facilities whenever possible.

- Establish a green hotel accreditation programme and leverage the Ministry of Tourism’s existing accommodation guide to review hotels based on these standards.

- Implement a revolving loan programme through the Tonga Development Bank to assist private entities in financing EE and net-metering projects.

- Institute an integrated resource planning process with TPL to identify how EE and renewable targets will be met in specific detail. Use this planning process to identify the cost effectiveness of other potential options, such as refurbishing or replacing diesel generators to increase efficiency.

## Combined wedge analysis

When combined, the suite of policy options from electricity and transportation offer a 50% reduction of CO2e a year by 2030. This represents a  reduction from the BAU emissions of 210,000 metric tonnes of CO2e, using 2020 as the refence year. Action taken in the ground transportation field is responsible for 30.4% of the projected emissions reductions, renewable electricity is responsible for 40%, and EE in the electricity sector is responsible for 29.6% of the reductions. Figure A-2 in the Appendix contains a breakdown of reductions by sector and policy.

Figure 30: Simplified combined wedge analysis

## Energy flow diagrams of 2017 baseline and 2030 goal

The changes that these emissions-reducing policies and projects would have on the energy use and infrastructure of Tonga is best viewed through a Sankey energy flow diagram. Figure 31 illustrates the flow in 2030 if all goals are met. It should be noted that these Sankey diagrams in the TEEMP cover only the electricity and ground transportation sectors, i.e. only a portion of the energy used, and GHGs emitted, in Tonga. Therefore, the breakdown of sectoral fuel use does not compare directly, when looking at percentages, to Tonga’s INDC.

Figure 31: Tonga 2030 energy flows with goals achieved in the electricity and ground transportation sectors

The major changes between the two diagrams are the addition of large quantities of renewable energy that mostly go to the commercial and residential sectors, and the associated reduction in diesel use. While space cooling grows in the residential sector, the amount of growth is substantially less under the TEEMP than under the BAU scenario. Similar measures are also able to reduce the portion of Tonga’s electricity used to power space cooling. Biodiesel also emerges as a fuel source for transportation; while it would be possible to mix biodiesel into fuel for use in TPL’s generators, this has not been included in the model, due to limited knowledge of TPL’s generator fuel requirements. While a reduction in line losses is indicated, the impact of these efforts would most likely be more dramatic. Renewable projects are likely to be located closer to the load they serve, and thus less likely to be susceptible to substantial line losses.

## Energy intensity with TEEMP policy changes

While the energy intensity of Tonga is projected to rise at a linear rate under the BAU, the policies outlined in TEEMP stabilise Tonga’s reliance on imported fuel. Figure 32 shows the difference between these two scenarios. The TEEMP scenario continues to decrease Tonga’s energy intensity to around 160 GJ per dollar of GDP per capita. An increase in Tonga’s energy intensity represents additional reliance on foreign fuel imports to power Tonga’s economic growth. Energy efficiency measures like those detailed in the TEEMP reduce Tonga’s vulnerability to volatility in fuel markets or supply chain issues.

Figure 32: Energy intensity of Tonga following TEEMP policy implementation

Projects and policies to reduce GHG emissions

The next step along the path towards achieving the goals laid out in Section 5, and to further verify the feasibility of these goals, is to propose policies and projects that would achieve them, package them as a programme in a logframe matrix, prioritise them and estimate their costs.

It should be noted that the scope of this study is only the transportation and electricity sector. A large portion of Tonga’s total emissions are from industrial processes, the agricultural sector, and other non-energy sources. Emissions reduction potential likely exists in these sectors as well, and should be addressed when adopting a comprehensive plan to meet Tonga’s emission targets under their deposited INDC.

## Transportation policies and projects

Table 8 lists some transportation policies and projects to help achieve the goals laid out by the wedge analysis. These policies and projects are presented with key considerations regarding their implementation. They are presented  with preliminary assessments of the size of potential GHG reduction, upfront cost (many of their lifecycle costs are negative), time to implement, ancillary benefits (which frequently translate to popularity or political feasibility), and available support.  Assessments are generalised as green for good, yellow for average, and red for bad. In the event of a tie for the number of positive scores, priority is given to the ranking on the left. The top ten options are elaborated in the text below. It is, however, worth mentioning three key developments that would affect fuel use, efficiency and GHG emissions and would also have other significant positive impacts on the Tonga economy. These are:

- the switch to 10 ppm diesel fuel since 2018;

- the very high priority the current government gives to improving the roads throughout Tonga since it came to power in September 2019; and

- The construction in 2021 of the bridge across the lagoon, linking the capital and Folaha village.

Table 8: Policies and projects to reduce GHG emissions from ground transportation

- Use a platform such as NextBus to coordinate buses.

  - Trackable buses greatly reduce time wasted due to arrival-time unpredictability, especially on routes such as those in Tonga, where buses do not circulate frequently. They will reduce stress and lost time, and encourage more ridership. Census data indicate that a significant (and growing) portion of Tonga’s population owns a cell phone, which can be leveraged to provide transportation data to customers on demand.

  - Resource: How NextBus Works

  - Resource: Urban Bus Services in Developing Countries and Countries in Transition

Consideration: Tonga’s bus industry is fairly uncoordinated, although they do have an association. NextBus is better suited to a developed industry with better developed infrastructure such as roads, bus stops, etc. Perhaps Tonga could begin with something like Fiji’s system, where buses are registered for a certain route and a timetable and deliver a reliable service to the public. There is an opportunity to look at establishing a national bus company prior to adopting NextBus based on the appropriateness of the transport infrastructure and consumer awareness.

- Enact a safe bicycle passing law for drivers passing cyclists or pedestrians.

  - A minimum passing distance law sends a signal that pedestrians and bikers have the right to be on the road, and that it is the responsibility of drivers to avoid hitting them. Of course, the law is only effective if police actively enforce it. One study found that in a state with such a law, while violations were plentiful, enforcement was rare.

  - Resource: League of American Bicyclists – Model Safe Passing Law

  - Resource: Public Policies for Pedestrian and Bicyclist Safety and Mobility

Consideration:  Existing legislation provides for the safety of every user of the road, including pedestrians. The current width of roads do not permit allocating a particular lane for bicycles, buses, etc. other than the footpaths for pedestrians.

- Send a mechanic to HEV/EV maintenance training in Japan or New Zealand.

  - This facilitates the adoption of both HEVs and EVs. Toyota operates a training programme in Japan specifically for foreign nationals, and MITO is developing a similar technical programme in New Zealand. Scholarships could be available through Toyota, MITO or JICA to make this a low-cost option to ensure that Tonga has the technical capacity to repair and maintain new vehicle technologies.

  - Resource: Toyota Tajimi Service Center

  - Resource: MITO Automotive Electrical Engineering Program

Consideration: Given a small market with high staff turnover and loss of trained people due to migration, consider working with a local industry, e.g Toyota, to conduct local training for local  mechanics. The training would be conducted by accredited trainers teaching accredited courses. Training in NZ and Japan should be on specialised areas targeting sponsored participants with years of experience in the industry.

- Provide pedestrians (particularly school-children) and cyclists with safety reflectors and lights.

  - VKT will increase so long as pedestrians feel safer in cars than on foot. Travelling on foot at night is a particular concern (75% of US accidents resulting in a pedestrian casualty occur in the dark). Various pilot programmes to distribute reflectors or lights to school-children have proven effective. Studies identify that reflector vests are the most effective option for increasing the visibility of cyclists to drivers.

  - Resource: A Road Safety Manual for Decision-Makers and Practitioners (Pages 78-81)

Consideration: The safety of children and pedestrians is a very high priority of the Ministry of Police in enforcing the relevant legislation.  Pedestrian casualties at night due to absence of reflectors and lights are minimal. More and more footpaths are constructed in the town areas and are heavily used by people doing evening and early morning exercises.

- Install rumble strips and painted lines demarcating lane boundaries to increase pedestrian and biker safety.

  - As most countries develop, they initiate a negative feedback loop. Cars are prioritised, VKT increases, and pedestrian and biker safety decreases, which encourages travel by car instead of walking or biking. Tonga, therefore, needs to safeguard pedestrian and biker space before this feedback loop commences. Rumble strips are a relatively inexpensive and effective way to keep drivers off the shoulder of the road, providing pedestrians with a reassurance of safety. In areas where the road is too narrow, “sharrows” reserve space for bikers and increase driver awareness.

  - Resource: Decision Support Guide for the Installation of Rumble Strips on Non-Freeways

  - Resource: A Road Safety Manual For Decision-Makers and Practitioners (Page 28)

Figure 33: Pedestrians in Tonga with limited right-of-way, compared to a road with a designated bicycle/pedestrian lane

(Photos by Caley Johnson and Kevin Collington)

Consideration: As in #4 above, safety of children and pedestrians is a very high priority of the Ministry of Police in enforcing the relevant legislation.  Footpaths are the priority for pedestrians and they are currently being constructed in town areas and villages. Dedicated lanes need to be considered in light of the competing demands for parking spaces, as well as the practicality of enforcing and monitoring, given current capacity and technology.

- Limit idle time with the help of fleet partners and idle reduction technologies

- Fleets can greatly reduce their fuel and maintenance costs by reducing their idle time. Outreach and partnerships that help them realize these savings generally well accepted—especially when they are affiliated with schools since children are particularly susceptible to air pollution. In addition, many states and municipalities around the world successfully restrict idling due to air and noise concerns. Numerous technologies are available that allow vehicles to reduce their idle time while performing tasks required of the idling vehicle.

- Resource: SmartWay Verified List of Idling Reduction Technologies

- Resource: Idle Reduction for Long-Haul Trucks

  - Case Studies: Clean Cities Fleets Using Idle Reduction Technologies

Consideration: Traffic during peak hours for the drop off and pick up of students are getting longer and longer. Village school buses are currently being used and the use of buses would assist too.

- Set a vehicle registration fee and/or import tariff according to fuel consumption.

- People are most responsive to up-front costs when purchasing vehicles rather than lifecycle ownership costs. Registration fees and import tariffs can, therefore, be set to make efficient vehicles much less expensive than inefficient ones. Countries have implemented such registration fees that are based on CO2 emissions, fuel consumption, fuel economy, engine displacement, or vehicle size. Some have even implemented a “feebate” system that actually pays people that register the most efficient vehicles. The recommended registration policy for Tonga would be based on GHG emissions (which allows for alternative fuels) and would be constant across size categories (to avoid perverse incentives to increase size), with a “feebate” system, all of which are much more politically feasible.

  - Resource: Vehicle Registration Fees Implemented by Countries

  - Resource: Fuel Economy State of the World 2016

  - Tool: New Zealand Fuel Economy Rating Database

Consideration: Government services fees and tariffs are usually submitted by the government of the day, together with its annual budget to be approved by parliament. Regulations are subsequently amended to facilitate the implmentation of the changes.

- Blend 10% biodiesel into all diesel fuel.

  - As described in section 5.1, by 2030 this quantity of biodiesel would be supplied by a majority of waste grease, supplemented by about 25% waste grease. Ten per cent biodiesel is compatible with nearly all diesel vehicles in a warm climate and provides lubricity and cetane benefits. To take full advantage of its waste grease resources, Tonga would need to build biodiesel production facilities with a total of about onemillion litres per year capacity, likely divided between a few islands. The economics of such small-scale waste-grease-to-biodiesel production facilities can be favourable, depending on numerous factors.

  - Resource: Economics of Small-Scale Biodiesel Production

  - Recource: European Biodiesel Board

  - Resource: National Biodiesel Board

  - Resource: Philippines Shell Foundation Inc. Project COCONUT (they are working with farmers in the Philippines to improve coconut productivity for biodiesel).

Consideration: Baseline study did not provide some firm estimates on waste grease nor of sources such as copra oil.  While the technology may have worked in Europe and the Philippines, there is a need for some detailed studies of the proposal to provide clearer guidance to the way forward.

- Water taxi in the laguna (and some associated dredging)

  - Transporting passengers via water taxi can be more efficient than road transportation on an energy per passenger-kilometer basis, depending largely on the size and occupancy of the vehicles. Furthermore, the distance from Nuku’alofa to various towns lining the laguna is much shorter than the distance around the laguna on Taufa’ahau Road. Ancillary benefits include time and traffic reductions. However, the laguna would likely require dredging in some locations.

  - Resource: Sarasota Water Taxi Feasibility Study

  - Resource: Forget Flying Cars: We Need Floating Ones

Consideration: The proposed ADB-funded bridge across the lagoon linking the capital to Folaha village is linked to this proposal. Construction of the bridge is planned to begin in 2021.

- Build a strategic parking lot and bus stop at the intersection of Taufa’ahau Road and Loto Road in Pea.

  - Strategic parking lots are often located at bottlenecks, where traffic gets worse, or at confluences of roads. They provide good locations where people can park their vehicles and catch a bus or carpool into congested areas with limited parking. This intersection in Pea meets these requirements, with nearly 17,000 vehicles passing through it per day on their way to and from Nuku’alofa. Parking lots in this location could alleviate the need for additional parking in Nuku’alofa as vehicle ownership increases, and could enable the city to remain clean, spacious and pedestrian-friendly. This parking lot system also sets the stage for reducing the parking in Nuku’alofa through a new parking policy.

  - Resource: Washington DC Park & Ride System facilitates carpooling and mass transit

Consideration: Personal convenience and one’s own independence would be a challenge to this proposal. The proposed change would need time to perfect itself and public education would be needed to support public acceptance and participation.

- Introduce fuel economy and fuel cost labeling requirements.

  - Economics provides a powerful reason for people to purchase more efficient vehicles. However, this is not clear unless people know the fuel economy of vehicles they are considering purchasing. Stickers on vehicles at dealerships that clearly state the expected annual fuel cost of the vehicle further illustrate this point. Using fuel economy on the Japanese test cycle would be a good start, or combining test cycles from multiple countries the way New Zealand does. However, to maximise accuracy, Tonga could develop their own test drive cycle, perhaps with the DRIVE tool.

  - Example: Fuel Economy Labeling, Focus on non-EU Countries

  - Review: Evaluation of Vehicle Fuel Efficiency Labeling and Consumer Information Programs

Consideration: This is regarded as a low hanging fruit that would be easy to implement and monitor. A lot of Tongans who have visited NZ and Australia are quite familiar with this vehicle labelling.

## Electricity energy efficiency policies and projects

Table 9 details preliminary least-cost policy options to achieve energy efficiency targets in more detail. Policy options are grouped into three categories: low hanging fruit, which are inexpensive relative to the results they would deliver; energy transformation projects, which could require lengthy or expensive implementation processes; and incremental improvements, which are policies that would influence the development of infrastructure over a longer time period.

Table 9: Rubric of least-cost building/appliance energy efficiency policy options

Details and selected case studies for some of these policy options are listed below.

- Implement building standards for resilience and energy efficiency (e.g. passive ventilation and daylighting with appropriate external shading).

- As GDP per capita increases, an increasing amount of power is used to supply load that could be better satisfied by mindful construction practices, such as orientating windows to optimise passive ventilation, strategic shading, and insulating building shells if air-conditioning is to be installed. Additionally, reviewing building codes provides an opportunity to ensure that new construction is well-engineered to withstand the worsening effects of tropical cyclones. Tonga can learn best practices from international partners to develop standards that are both easy to interpret and consistent with UN Sustainable Development Goals 7, 9, and 11.

- Ongoing project: Review, strengthening and updating of the Tonga Building Code

- Example: Guidelines for cyclone resistant construction of buildings in Gujarat (India)

- Resource: Mainstreaming Building energy efficiency codes in developing countries

Consideration: This is a policy option that the Ministry of Infrastructure is currently working on, for instance, the review of the current building code.

- Implement a public awareness campaign on energy efficiency and conservation.

- Create an energy-efficiency public awareness campaign, focusing on both the environmental, and personal economic benefits to rate-payers who reduce their consumption. The introduction of an MEP Sprogram goes hand in hand with this; while 80% of Tongans said appliance running costs are important to them, only 34% plan on looking for an AUS/NZ MEPs label next time they are purchasing an appliance. Introducing a programme that is easy to understand and educating customers as part of a broader energy efficiency public awareness campaign is critical. Customs agents need to be empowered to halt any imports of inefficient appliances and establish packaging and recycling standards to ensure that any public awareness campaign is backed by meaningful action.

- Example: Raising awareness of energy efficient light bulbs pays off in Rwanda

- Resource: Creating an awareness campaign

Consideration: The Pacific Appliance Labelling and Standards (PALS) programme has been successfully implemented throughout the Pacific Islands. The proposed legislation regarding MEPS is incorporated into the draft Tonga Energy Act. Energy stakeholders such as the MEIDECC, TPL and the MTED all have public awareness programmes that need support to become more targeted and more effective.

- Establish a demand-side management revolving loan or rebate programme to aid in financing more efficient equipment (residential, commercial and industrial).

  - With a small initial investment – potentially funded in part by development banks – the existing capabilities of the Tonga Development Bank could be leveraged to provide ultra-low interest loans to projects that demonstrate they can generate energy savings. These savings are used to pay off the full amount of the loan in a five- to ten-year period, at which point the fund is redistributed to subsequent projects. Rebate programmes similarly encourage customer uptake of efficient technologies.

  - Example: The Thai Energy Efficiency Revolving Fund

  - Resource: Exploring the case for revolving funds for domestic energy efficiency programs

Consideration: There are existing experiences such as a revolving loan with the Tonga Development Bank, supported by the IUCN. There are also government funds within TDB for targeted policy initiatives of government, such as supporting agricultural exports, increasing tourism earnings, etc.

- Work with TPL to create an integrated resource plan to incorporate RE, EE and more efficient reciprocating engines that can be dual fuel.

  - While TPL is eagerly working towards Tonga's renewable energy goals, the company publishes little long-term planning, detailing how these goals will be achieved. An integrated resource plan provides an opportunity for the company to lay out a long-term, realistic framework, outlining how goals will be met in a scope not captured by annual reports. They can use modeling such as PLEXOS or SAM to integrate cleaner energy generation sources to provide peaking power, and limit Tonga’s exposure to fluctuations in the price of diesel.

  - Example: Hawaiian electric companies IRP

  - Resource: Greening the grid—grid integration studies

  - Resource: Best practices in electric utility integrated resource planning

Consideration: There are current efforts to review Tonga’s NDC and to derive an NDC investment plan. All of these would provide inputs to the development of a TPL IRP.

- Perform energy audits of buildings to create baselines and implement energy conservation measures.

  - According to one study, up to 3.5% of total consumption on Tongatapu originates from a Church of Latter-Day Saints temple, most likely a result of superfluous air-conditioning. Strengthening energy efficient building standards and performing audits on the largest consumers – including government buildings – can play a large role in reducing overall demand.

  - Example: Caribbean Hotel Energy Efficiency Action Program

  - Resource: A Guide to Energy Audits

Consideration: This is an area where  close collaboration between the MEIDECC and the TPL will be required.

- Establish a data collection exercise/database to manage energy data by sector.

  - Work with TPL, the Bureau of Statistics, the MOI, and the Public Service Commission to standardise annual reports, and increase reporting of data to include figures such as sectoral consumption and load profiles by source of generation.

  - Example: Fiji Electric Authority, 2016 Annual Report

  - Resource: Energy analytics for development

Consideration: There are various data collection efforts going on. At the regional level is the Pacific Regional Data Repository. Tonga has been a beneficiary of ESCAP’s Evidence-based Policies for the Sustainable Use of Energy Resources in Asia and the Pacific, and there are current collection efforts by TPL and the Bureau of Statistics. The draft Energy Act has clauses to support more effective and coordinated energy data collection and sharing among the energy stakeholders.

## Resilience considerations

Tonga needs electricity and transportation systems that are resistant to damage from cyclones and can recover quickly from them. Many of the projects proposed in previous sections can enhance resilience if steps are taken to ensure that projects or policies add to the systems’ resilience to cyclones and other natural hazards.

Tonga’s electric and transportation system can be made more resilient by following the basic steps listed below, which are tied to the policies and projects above.

- Incorporate resilience into building codes. Many features that support energy efficiency in buildings also contribute to cyclone resilience. Examples of efficiency and resilience include: planting trees and building verandas which provide natural shade and also buffer wind gusts; ensuring that such objects are close enough to provide shade, but far enough away to avoid presenting a risk of damaging buildings or other infrastructure; installing cool roofs with cyclone resistant materials, ensuring the use of hardened materials that also enhance insulation levels to reduce energy needs; and incorporating building designs (as shown in Figure 8) that shed wind loads.

- Continue to harden T&D lines and power generation. TPL staff should continue to work with the Government of Tonga to bury lines in cyclone prone areas in order to reduce the number of aerial lines with the potential for wind damage, as is currently being undertaken by the NZ-funded Tonga Village Network Upgrade Project. Prioritising areas that are frequently disrupted may help with strategic planning. Installing microgrids with onsite renewable energy technologies in key areas to reduce the need for centralised power, raising/elevating substations or equipment in flood-prone areas, and moving assets away from areas susceptible to storm surge or sea-level rise can all be beneficial techniques to hardening the generation and distribution system for more reliable power.

- Fuel diversification. More fuel sources increase the likelihood that portions of a transportation system will be able to function even if the main source of fuel (petroleum from Singapore) cannot be accessed. The proposed projects that do this for Tonga are biodiesel and electric vehicles (with electricity coming from renewables rather than from petroleum). Renewable natural gas (RNG) from farms, landfills and sewage treatment facilities are another option that was not proposed in this plan due to budget considerations but could be favourable if Tonga places high value on the resilience benefits this fuel offers.

- Improve vehicle efficiency. In disaster scenarios, efficient vehicles can get more work done with a limited amount of fuel. A vehicle import policy requiring more efficient vehicles would then make Tonga’s ground transportation system resilient to disaster-caused fuel limitations.

- Reduce dependence on roads. Cyclones can render roads impassable by blowing debris onto them or washing water over them. Therefore, any project that reduces the distance needed to travel on roads in order to perform basic transportation functions will make the system more resilient. One such project is the proposed water taxi system in the Laguna. These water taxis do not rely on roads and could provide invaluable service when the roads are blocked. Another such project is the bridge over the north side of the Laguna. As can be seen in Figure 19, such a bridge could enable traffic from the east side of Tongatapu to reach Nuku’Alofa, even if the Taufa’ahau and Liku roads are blocked. Even if these roads were passable, the distance and fuel that this bridge could save would be even more important in times when fuel is limited due to a cyclone.

- Optimise the use of appropriate vehicles. When roads are flooded and covered with debris, regular cars only further clutter the roads. Therefore, it is good to replace them with vehicles that have higher clearance and can carry more passengers. Buses have these characteristics, and two proposed projects can help make them more effective during cyclones. First, a software platform such as NextBus makes any given bus more effective at passenger transport by making it more trackable, especially in situations where the usual bus routes and timing need to be changed. Secondly, the strategic parking lot at the intersection of Taufa’ahau and Loto Roads could make evacuation procedures easier during/after a cyclone by keeping them from stalling and clogging the roads of Nuku’alofa.

5.4 Gender and income equality considerations

Tonga’s power system transformation provides a unique entry point for additional female participation in Tonga’s workforce, particularly in science, technology, engineering, and math (aka STEM) fields. This transformation could also reduce the cost of electricity for all Tongan households, which will be especially useful for lower-income or single-parent households with constrained budgets. Tonga, like much of the world, has a lower female than male workforce participation rate due to traditional role fulfillment of women as homemakers. In 2016, 26.8% of Tongan women described themselves as unemployed but willing to begin work (compared with 7.6% of men), but only 1.4% of women are unemployed and actively seeking work (compared with 0.7% of men). This gap of Tongan women who are willing and available to begin work, but are not actively seeking work persists, despite the fact that Tongan women are more likely to hold tertiary education (11.8% vs 9.8% of men).69 It is likely that a number of educated women choose to work in the traditional role of housemaker, despite a desire and ability to work outside the home. Creating domestic energy jobs opens the door for these women to persue a career, and could benefit Tonga’s overall economy by improving labour force participation rates. The suite of policy proposals in the TEEMP provides opportunities for Tonga to provide domestic investment in RE, transportation sector transformation, and EE. Tecnicians, installers, auditors and analysts are all new positions that will be created and could be fulfilled by Tongans of both sexes. The TEEMP needs to be implemented with these statistics in mind – efforts should be made to encourage gender parity in hiring for these new positions. Additionally, reducing local pollution created by 
diesel generators and vehicles creates better health outcomes, particularly for vulnerable populations such as the 
elderly, pregnant women, and children.

At the consumer level, spending on fuel accounts for 6% of Tongan household expenditure, and spending on
electricity accounts for 3%.70 For rural households, households with a high number of occupants, or households with limited income (or only a single provider of income), the cost of energy likely makes up a significantly higher proportion of household expendenture. The Gini coefficient is a good measurement of economic inequality within a country’s economy. Tonga has a moderate Gini coefficient of 0.417 nationally (measured as a decimal value between 0 and 1), but it is more pronounced in urban areas of Tongatapu (0.444).71 Access to clean and resilient energy sources, such as those provided by solar or wind resources, is increasingly seen as a guaranteed human right by international organisations, and Tonga has reaffirmed its commitment to this tenet through its ardent participation in the UN’s Sustainable Development Goals. Failures in the electrical grid due to storms, drastic variations in fuel or electricity prices, and fuel shortages all threaten universal access to these resources. Tonga’s exposure to energy shortages requires that particular attention be paid to the nexus of income level, gender balance, and energy security. Across Tonga, 44% of households own a generator – presumably most households keep it for backup power. Although these households have access to independent electricity production in the event of grid outages, it is the responsibility of Tongan officials and TPL to ensure that resilient electricity is delivered to the 56% of Tongans who might not be able to afford a backup generator.72 By fostering an environment of energy independence and resilience, Tonga is ensuring equitable energy security for all of its residents.

Monitoring the TEEMP

The TEEMP should be viewed as a national programme that tends to coordinate Tonga’s EE effort, beginning with the electricity and transport sectors, but which other sectors such as agriculture and waste can be added once developed and studied.

The general objective of the programme should be consistent to the TERM’s as Reduce Tonga’s Vulnerability to Oil Price Shocks and Achieve an Increase in Quality Access to, and use of, Modern Energy Services in an Environmentally Sustainable Manner.

## The purpose of the TEEMP is to reduce GHG emissions from both transportation and electricity by 50% from the BAU by 2030.

For the monitoring of the TEEMP, there are four suggested results:

Result 1: Improved institutional, policy, legislative and governance frameworks for coordinating and managing the implementation of the TEEMP;

Result 2: Improved awareness and funding support and strengthened local technical capacity to plan, implement and monitor low carbon projects in the transportation and electricity sectors;

Result 3: Reduced greenhouse gas emissions in the transportation sector; and

Result 4: Reduced greenhouse gas emissions in the electricity sector.

.

Given that the TEEMP is part and parcel of Tonga’s TERM, and hence the adoption of the TERM objective to be used for the TEEMP, the management and M&E of the TERM is to be conducted by the same institutional structure as the TERM, with the inclusion of the Ministry of Agriculture and the Waste Authority.

Table 10 is a complete logframe matrix for the TEEMP.

Table 10:  Logical framework for the Tonga Energy Efficiency Master Plan  (2020–2030)

|  | Intervention logic | Objectively verifiable indicators of achievement | Sources and means of verification | Assumptions |
| --- | --- | --- | --- | --- |
| General objective | The general objective is to reduce Tonga’s vulnerability to oil price shocks and achieve an increase in access to, and use of, modern energy services in an environmentally sustainable manner | - % growth rate of Tonga’s dependence on fossil fuel is reduced by 2030 - % improvement / increase in Tonga’s energy intensity by 2030 - 70% of the electricity generation is from RE by 2030 | - Annual Report of MEIDECC - Annual Report of TPL - Progress reports of the TERM Plus & NDC | - Current and subsequent govts are committed to the Paris Agreement |
| Project purpose | ## The purpose of the TEEMP is to reduce GHG emissions from both transportation and electricity by 50% from the BAU by 2030. | - A reduction of at least 106,000 metric tonnes of CO2 equivalent per year by 2030 in both transport and buildings | - Annual Report of MEIDECC - Progress report on the NDC | - Current and subsequent govts are committed to the TEEMP |
| Results | Result 1: Improved institutional, policy, legislative and governance frameworks for coordinating and managing implementation of the TEEMP | - The Tonga Energy Efficiency Regulation on MEPSL becomes a part of the Energy Act by 2025 | - Report of the Attorney General’s Office & minutes of Cabinet meetings | - Current and subsequent govts are committed to the reform of the energy sector |
|  |  | - The Building Code is reviewed to incorporate EE and resilience by 2023 | - MoI annual report | - MoI support |
|  |  | - The TERM institutional structure is activated in 2020 to coordinate and manage the implementation, monitoring and reporting of the TEEMP | - Cabinet decision | - MEIDECC support |
|  | Result 2: Improved awareness and funding support and strengthened local technical capacity to plan, implement and monitor low carbon projects in the transportation and electricity sectors | - At least one international and one local launch of the TEEMP by end of 2021 | - MEIDECC launch reports | - Covid-19 travel restrictions are lifted. |
|  |  | - 50% of the TEEMP budget gets confirmed funding by 2025 | - MEIDECC launch reports - Ministry of Finance Budget Reports | - TEEMP is featured in Tonga LEDS, GCF Country Programme, NDC Investment Plan, etc. |
|  |  | - 20% of the TEEMP budget comes from the National Budget by 2025 | - MEIDECC budget - Ministry of Finance Budget Reports | - Cabinet support |
|  |  | - TEEMP gets promoted in at least five investment forums by 2024 | - Annual reports of the MEIDECC | - MEIDECC support |
|  |  | - 200 people trained on scholarships and on accredited energy auditing courses in transportation and electricity by 2025 | - Annual reports of the MEIDECCC and Ministry of Education and Training | - TNQAB support |
|  |  | - EE is incorporated in the primary, secondary and TVET school syllabuses by 2025 | - Annual reports of the MEIDECC and Ministry of Education and Training | - MET support |
|  |  | - An accredited national qualification on EE is adopted by the TNQAB by 2025 | - Annual reports of the MEIDECC and Ministry of Education and Training | - TNQAB support |
|  |  | - No. of trade fairs completed & No. of public appearances | - MEIDECC reports | - MEIDECC support |
|  | Result 3: Reduced greenhouse gas emissions in the transportation sector | - 20% reduction in VKT of light duty vehicles through walking, biking, public transit, ridesharing and telecommuting by 2023 | - Tonga National Communication & reports on the progress with its NDC implementation. | - MoI support |
|  |  | - HDV idle time reduced by one hour per day by 2030 | - MoI vehicle registration | - MoI support |
|  |  | - All road transportation diesel contains 10% biodiesel by 2030 | - MoI vehicle registration | - Biodiesel production is economically feasible |
|  |  | - 10% of new LDVs are hybrid and electric by 2030 | - MoI vehicle registration | - Adopted tax incentives on e-vehicles |
|  |  | - 20 GPS fish locator and VMS deployed in fishing by 2024 | - Ministry of Fisheries reports | - MoF support |
|  | Result 4: Reduced greenhouse gas emissions in the electricity sector | - At least 10 major electricity consumers are audited and recommendations are implemented by 2026. | - AMREC and contract reports | - TPL and MEIDECC support to the AMREC |
|  |  | - Three additional appliances are included in the MEPSL regulation by 2026 | - Regulations and government gazette | - MEPSL continues to be implemented |
|  |  | - At least a 5% decrease in line losses and a 5% increase in underground cables by 2030 | - TPl reports | - Continuation of the Tonga Village Network Upgrade project |
|  |  | - At least 6 MW of RE mini-grids plus battery storage are installed by 2030 | - TPL & MEIDECCC reports | - Favourable investment climate |
|  |  | - 100% of all street lights and government premises are on LED lights by 2030 | - AMREC & MoI reports | - Government’s support |
|  |  | - 100% of the tourism accommodation premises comply with the hotel guide by 2030 | - Ministry of Tourism reports | - MoT support |
|  |  | - At least USD 5 million is secured for an EE revolving fund / rebate by 2030 | - Energy sector reports | - Donor support |
|  |  | - An effective public awareness programme reach out to 90% of the population by 2027 | - Media reports | - MEIDECC and TPL have dedicated comms staff |
|  |  | - At least USD 2 million is secure to kick-start a cool roof programme by 2027 | - Energy sector reports | - Donor support |
|  |  | - An IRP for TPL and a data base programme for the energy sector is adopted by 2025 | - TPL and statistics reports | - TPL Board’s support. |

Financing the TEEMP

It is estimated that the financing of the TEEMP would require about USD 27 million, with 7% or USD 1.97 million to implement low hanging activities within its first three years. The bulk of the financing (87% or USD 23 million) would be needed for the more difficult and longer term activites in years 4-7 while 6% or USD 1.5 million would be required for the implementation of the least urgent activities in years 8–10.

Activities have been grouped based on a preliminary assessment of the ease by which they can be implemented and financed, given the very crude nature of of the GHG emission estimates.

Tonga’s GCF country programme includes reducing emission from the land transportation (energy efficiency) with the JNAP 2, NDC and the TERM as sources of strategic priority. There is an estimate of USD 12 million for this priority activity, which would be more than sufficient to cover the estimated costs under transport in the TEEMP. Tonga’s other multilateral and bi-lateral partners are potential sources of financing the TEEMP.

The Government of Tonga can also contribute to the TEEMP through its funding of the MEIDECC staff and their operational costs. With the necessary promotion and incentives, the private sector could find it attractive to invest in the activities of the TEEMP.

There are parallel activities that are related to the financing of the TEEMP. Together with the GGGI, Tonga is developing its Low Emission Development Strategies (LEDS) and with the GGGI and the Pacific NDC Hub it is reviewing its  NDC and developing an NDC Invesment Plan. These initiatives will be complementary to the TEEMP and vice versa.

Table 11 is a draft of the key activities under each results area of the TEEMP, together with a rough estimate of the costs and a grouping of the activities according to how easily they can be funded and fully implemented.

Table 11: Key activities under each result area

| ACTIVITIES | MEANS | COSTS (IN USD) | RESPONSIBLE LEAD AGENCIES | Modelled GHG Impacts (annual CO2e reduction by 2030) | Year 1–3 | Year 4–7 | Year 8–10 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | High | Medium | Low |
| Result 1: 1.1 Draft and conduct consultations on an MEPL regulation to be added to the Energy Bill | Consultancy, workshops and meetings, training and awareness, etc | 300,000 | MEIDECCC |  | 300,000 |  |  |
| 1.2 Review the Building Code to incorporate EE by 2023; seek Cabinet approval and support its associated awareness and enforcement activities | Consultancy, workshops and meetings, training and awareness, etc | 200,000 | MoI |  |  | 200,000 |  |
| 1.3 Reactivate the TERM institutional structure in 2020 to coordinate and manage the implementation, monitoring and reporting of the TEEMP | Cabinet submission and consultation meetings | 5,000 Sub-total: 505,000 | MEIDECCC |  | 5,000 |  |  |
| Result 2: 2.1 Launch of the TEEMP locally and at COP 26 on 9–19 November 2020, in Glasgow, UK | Travel, publications, promotional materials, venue, catering & equipment | 100,000 | MEIDECCC |  | 100,000 |  |  |
| 2.2 Conduct annual energy investment forums and other relevant forums to promote the TERM+, TEEMP, etc | Travel, publications, promotional materials, venue, catering & equipment | 500,000 | MEIDECCC |  |  | 500,000 |  |
| 2.3 Offer scholarships and conduct accredited energy auditing course in transportation and electricity and implement the findings | Scholarships, trainers, training materials, tools & equipment, venue, catering | 250,000 | AMREC supported by MEIDECCC & TPL |  |  | 250,000 |  |
| 2.4 Incorporate EE in the primary, secondary and TVET school syllabus | Consultancy, workshops, training of trainers, teaching materials and equipment | 300,000 | MET |  |  | 300,000 |  |
| 2.5 Develop a national and accredited qualification on EE and an energy literacy programme for all sectors | Consultancy, workshops, training of trainers, teaching materials and equipment | 300,000 | MET |  |  | 300,000 |  |
| 2.6 Promote local funding for the TEEMP during the budget preparation process | Promotional materials, meetings, displays and site visits | 10,000 Sub-total: 1,460,000 | MEIDECCC |  | 10,000 |  |  |
| Result 3: 3.1 Implement a programme (including physical structures) to promote the safety and health benefits of walking and cycling | - Consultancy equipment, training and workshops - Enact safe bicycle passing law - Distribute safety reflectors and lights - Install rumble strips | 400,000 | MoI & MET | - 12,700 - 12,700 - 12,700 |  | 400,000 |  |
| 3.2 Implement a programme to improve efficiency of buses and limit vehicle idle time | - Consultancy equipment, training and workshops - Study and establish national bus company - Increase number of school buses - Use platform for coordinating buses - Build strategic parking lot and bus stops at bottlenecks - Introduce technologies to reduce idle time | 500,000 | Chamber of Commerce, MoI & School PTAs | - X - X - 12,700 - 12,700 - 10,300 |  |  | 500,000 |
| 3.3 Introduce water taxis in the Fanga’uta lagoon | Consultancy, dredging, testing & promotion | 250,000 | MoI | 12,700 |  |  | 250,000 |
| 3.5 Adopt appropriate fiscal and financial incentives to promote HEVs, EVs and fuel economic vehicles, tuks-tuks, etc. | Consultancy, workshops, training and demonstration | 200,000 | MEIDECCC & Finance and National Planning |  |  | 200,000 |  |
| 3.6 Promote biodiesel & e-mobility | - Consultancy testing 10% biodiesel, demonstratio, equipment and training - Training of trainers on e-mobility | 350,000 | MoI & Agriculture | a. 4,500 b. 300 |  | 350,000 |  |
| 3.7 Set vehicle registration and import tariff according to fuel consumption, engine size, and improve customer awareness | - Consultancy, consultation meetings and workshops - Introduce fuel economy labels, consultancy, training and consultation workshops | 250,000 Sub-total 1,950,000 |  | - 5,700 - 5,700 Total for Transport 102,700 | 250,000 |  |  |
| Result 4: Activities 4.1 Implement building standards (see 1.2). | Consultancy, workshops, training, equipment, demonstration, etc | 300,000 | MoI | 1,100 | 300,000 |  |  |
| 4.2 Conduct energy audits and implement the recommendations | Consultancy, workshops, training of trainers, equipment, demonstration, etc | 500,000 | AMREC supported by TPL and MEIDECC | 700 |  | 500,000 |  |
| 4.3 Extend the PALS programme to include other appliances and promote the use of solar water heaters | Consultancy, workshops, training, equipments, demonstration, etc | 250,000 | MEIDECCCChamber of Commerce, Consumer Division MTED | 10,100 | 250,000 |  |  |
| 4.4 Reduce TPL line losses to the identified optimum level, implement grid-hardening measures & extend the Village Network and the underground cabling works | Consultancy, workshops, training, equipment, demonstration, etc | 10,000,000 | TPL |  |  | 10,000,000 |  |
| 4.5 EE Public awareness campaigns | Promotional materials, publicity and visibility events and promotions, transport, etc | 250,000 | MEIDECCC, TPL, NGOs | 600 | 250,000 |  |  |
| 4.6 Install mini-grid projects and RE systems with storage | Consultancy, equipment, training, meetings and workshops, awareness and promotion | 10,000,000 | TPL & MEIDECCC |  |  | 10,000,000 |  |
| 4.7 Change all streetlights and lights at government premises to LED | Equipment, training, demonstration, awareness and visibility, etc | 500,000 | AMREC |  | 500,000 |  |  |
| 4.8 Implement a certification process for sustainable / green hotels by year 7 | Consultancy, equipments, trainings, meetings and workshops, awareness and promotion | 200,000 | Ministry of Tourism |  |  | 200,000 |  |
| 4.9 Implement a cool roof programme | Consultancy, equipment, training, meetings and workshops, awareness and promotion | 250,000 | MoI |  |  |  | 250,000 |
| 4.10 Establish a DSM revolving loan or rebate programme by 2030 | Consultancy, training, meetings and workshops, awareness and promotion | 500,000 | Banks |  |  |  | 500,000 |
| 4.11 Strengthen the planning and data collection and analysis capacity in the energy sector | Consultancy to draw up a IRP for TPL, workshops, equipment, training | 250,000 Sub-total 23,000,000 GRAND TOTAL 26,915,000 | TPL & MEIDECCC | Total for electricity 12,500 TOTAL 115,200 | 1,965,000 | 250,000 23,450,000 | 1,500,000 |

- 8 Conclusion

Through the outlined suite of policy options, a reduction of 50% a year by 2030 is
possible, using 2020 as the reference year. That represents a reduction from the BAU emissions of 210,000 metric tonnes of CO2e. Action taken in the ground transportation field is responsible for 30.4% of the projected emissions reductions (33,400 metric tonnes CO2e); renewable electricity is responsible for 40% (43,500 metric tonnes CO2e); and EE in the electric sector is responsible for 29.6% of the reduction (32,500 metric tonnes CO2e).

These reductions will have three primary effects: lowering emissions, improving climate resilience, and improving
energy independence. Although Tonga’s NDC does not lay out specific emissions reduction goals, ambitious
national goals target a high renewable penetration. By engaging in EE measures, total capacity requirements are
reduced, and the cost associated with new generation declines. The best way to reduce emissions is to reduce load,
and so it makes sense to harmonise Tonga’s ambitious renewable goals with equally ambitious EE goals. Secondly,
recent history has shown that tropical cyclones and other natural disasters can have devastating effects on small
island developing states. Tonga has intimate experience of this and has voiced strong support for polices to curb
climate change at the international level. Despite these valiant efforts, damage to the earth’s climate presents
immediate risks to Tonga’s infrastructure and power grid. Many of the EE projects outlined by the TEEMP work to
improve Tonga’s climate resilience by reducing unnecessary grid and road load, distributing power generation,
diversifying the fuel mix, and strengthening vulnerable infrastructure, while identifying opportunities for EE
improvements. Clearly, mainstreaming climate resilience is deeply integrated in the TEEMP. Finally, by increasing
renewables penetration and decreasing fuel consumption from the BAU scenario, Tonga’s dependence on imported
fuel – as measured by energy intensity – stabilises. Volatile fuel prices need not affect Tonga’s cost of electricity,
transportation, or cost of living; this results in better public welfare outcomes.

The next steps to achieve these outcomes should be coordinated through an institutional set up, similar to the TERM’s,  headed by MEIDECC, and members should include MOI, TPL, MCCTIL, PCREEE, Agriculture and Waste, the Department of Statistics, and other Tongan organisations. A memorandum of understanding for data-sharing and other coordination would need to be signed early in the process. The working group should strengthen ties with international organisations and financing partners that have demonstrated a strong interest in Pacific Island data, and with regional organisations, such as the UN ESCAP, PRDR, the Green Climate Fund, the Asian Development Bank, and the CTCN. It should also promote private sector investments in the TEEMP. The working group and international partners could then ensure that necessary data are centrally reported and stored. They could then implement EE and transportation efficiency improvements like those outlined in this report, alongside infrastructure and RE developments. Such cooperation and progress is key to ensuring that Tonga’s power and transportation system transformation is manageable and capable of meeting the emission reduction and energy independence goals laid out in the TERM in an expedient, organised, and cost-efficient manner.

- Appendix A. Supporting documentation

# Appendix Figure 1: Yearly A1 jet fuel consumed in Tonga (Source: MTED fuel volume 2012–2015 report)

# Appendix Figure 2: Tonga CO2E emissions reductions with sectoral breakdown

- Endnotes

“Tonga Energy Road Map 2010-2020” (Government of Tonga, June 2010), 3, https://sustainabledevelopment.un.org/content/documents/1330tongaEnergy%20Strategy.pdf.

Ibid.

Ibid, 8.

Government of Tonga, “Electricity Act” (2007), http://prdrse4all.spc.int/system/files/cap-32.12-electricity-act.pdf.

Government of Tonga, “Renewable Energy Act” (2008), https://ago.gov.to/cms/images/LEGISLATION/PRINCIPAL/2008/2008-0010/RenewableEnergyAct2008_1.pdf.

“Joint National Action Plan on Climate Change Adaption and Disaster Risk Management 2010-2015” (Ministry of Environment and Climate Change and National Emergency Management Office, July 2010), http://www.sprep.org/att/IRC/eCOPIES/Countries/Tonga/66.pdf.

Government of Tonga, “Ozone Layer Protection (Amendment) Act of 2014,” Pub. L. No. Act 2, accessed March 12, 2018, http://macbio-pacific.info/wp-content/uploads/2017/08/21A-OzoneLayerProtectionAmendmentAct2014.pdf.

“Tonga Village Network Upgrade Project – Reduces Line Loss and Elevates Power Distribution,” Tonga Power Limited, accessed March 12, 2018, http://www.tongapower.to/NewsRoom/tvnup_update_dec2012.aspx.

“Tonga, Phase 2 Presentation” (Promoting Energy Efficiency in the Pacific, International Institute for Energy Conservation), accessed February 6, 2018, http://prdrse4all.spc.int/system/files/peep2_-_country_presentation_tonga.pdf.

“Tonga Strategic Development Framework 2015-2025” (Government of Tonga, May 2015), http://www.finance.gov.to/sites/default/files/TSDF%20II_English_printed%20to%20LA%20on%2019May2015_0.pdf.

“OIREP Invitation for Bids,” Outer Island Renewable Energy Project, February 23, 2017, http://www.gov.to/wp-content/uploads/2017/03/1S1E-ICB-TPL-Vav-06-Invitation-for-Bids.pdf.

“Tonga Renewable Energy Project (TREP), under the Pacific Islands Renewable Energy Investment Program” (Asian Development Bank, March 17, 2017), https://www.greenclimate.fund/documents/20182/893456/16450_-_ADB_-_Tonga_Renewable_Energy_Project__TREP___under_the_Pacific_Islands_Renewable_Energy_Investment_Program.pdf/76c24e30-4a86-4369-b9a8-7f699d77cf2c.

University of Texas, available online at https://legacy.lib.utexas.edu/maps/islands_oceans_poles/tonga.jpg.

“Annual Report” (Tonga Power Limited, 2014), 5–6, http://www.tongapower.to/Portals/2/Docs/AnnualReports/TPL%202014%20Annual%20Report.pdf..

“TPL Tongatapu & ‘Eua Distribution Network Restoration Plan” (Tonga Power Limited, 2018).

“2017–2018 Tropical Cyclone Season Outlook for Tonga,” ReliefWeb, October 25, 2017, https://reliefweb.int/report/tonga/2017-2018-tropical-cyclone-season-outlook-tonga-0..

“Pacific Region Takes Spotlight at COP23,” Fiji Times Online, accessed February 22, 2018, http://www.fijitimes.com/story.aspx?id=423172&.

“Tonga 2016 Census of Population and Housing” (Tonga Statistics Department, October 2017), https://spccfpstore1.blob.core.windows.net/digitallibrary-docs/files/07/07f43535eac99827825b1da8e2ee750c.pdf?sv=2015-12-11&sr=b&sig=dUQz%2FuwGZff6%2F763deG2siKIWj0xFKsnoOG9ijCV%2FvQ%3D&se=2018-12-17T02%3A48%3A42Z&sp=r&rscc=public%2C%20max-age%3D864000%2C%20max-stale%3D86400&rsct=application%2Fpdf&rscd=inline%3B%20filename%3D%22Tonga_2016_Census_Report_Vol_1.pdf%22

“Progress Toward Sustainable Energy Global Tracking Framework 2015” (PRDRSE4All, July 2016). http://prdrse4all.spc.int/data/progress-toward-sustainable-energy-global-tracking-framework-2015-gtf-2015

Calculated from 2016 and 2017 TPL Annual Reports.

Argonne National Laboratory. 2017. The Greenhouse gases, Regulated Emissions, and Energy use in Transportation (GREET) Model.

“GREET Emissions Database,” (Argonne National Laboratory, 2017) https://greet.es.anl.gov/

“Tonga Country Energy Security Indicator Profile 2009,” (Secretariat of the Pacific Community, Euei) https://issuu.com/jale-s/docs/tonga_country_profile

“Tongatapu Monthly Usage,” accessed March 13, 2018, http://www.tongapower.to/Customers/MonthlyUsuage/TongatapuMonthlyUsage.aspx.

“Kingdom of Tonga: Electric Supply System Load Forecast” (Asia Sustainable and Alternative Energy Program, March 2010), 19, https://www.astae.net/sites/astae/files/publication/Tonga-Electric-Supply-System-Forecasts%5B1%5D.pdf.

“Building and Service Control Division Website” (Tonga Ministry of Infrastructure) http://infrastructure.gov.to/building-control/

“National Institute For Building Sciences Releases New Report,” (NIBS) https://www.nibs.org/news/381874/National-Institute-of-Building-Sciences-Issues-New-Report-on-the-Value-of-Mitigation.htm

“Deltec’s Hurricane Proof Homes Weather the Storm,” (Ecogen Pulse) http://www.ecobuildingpulse.com/projects/deltics-hurricane-proof-homes_o

“Electrical Appliance Energy Labelling and Minimum Energy Performance Standards for Pacific Island Nations” (George Wilkenfeld and Associates, December 1996), 
https://s3.amazonaws.com/clasp-siteattachments/1996-12_PacificIslandLabelsMEPS.pdf

“Energy Labelling and Minimum Energy Performance Standards for Appliances and Lighting: Impacts in Cook Islands, Fiji, Kiribati, Samoa, Tonga and Vanuatu” (Austalian Aid, 2016), http://prdrse4all.spc.int/sites/default/files/energy_labelling_and_minimum_energy_performance_standards_for_appliances_and_lighting-impacts_in_ci.fj_.ki_.sa_.to_.va_.pdf

Tebbutt Research, “Survey of Consumer Awareness and Use of Energy Rating Labels in PICs: Tonga Country Report” (The Pacific Commuity)

Lucas Davis and Paul Getler. “Contribution of air conditioning adoption to future energy use under global warming,” PNAS. December, 2014. http://www.pnas.org/content/pnas/early/2015/04/22/1423558112.full.pdf

“Electrical Appliance Energy Labelling and Minimum Energy Performance Standards for Pacific Island Nations” (George Wilkenfeld and Associates, December 1996), 
https://s3.amazonaws.com/clasp-siteattachments/1996-12_PacificIslandLabelsMEPS.pdf

U.S. Department of Energy software, eQUEST. Available online at http://www.doe2.com/eQUEST/

Yarmouth, Massachusetts lighting project information available at https://www.yarmouth.ma.us/1247/LED-Streetlight-Project

Liisa Ecola et al., “The Future of Driving in Developing Countries,” RAND Corporation, n.d., 138.

National Reserve Bank of Tonga, “Vehicle Registrations,” accessed March 19, 2018, http://reservebank.to/index.php/economic/vehicle-registrations.html.

Ministry of Merchandise and Trade Statistics, “Foreign Trade Reports,” 2008-2016.

International dollars, also known as Geary-Khamis dollars, are based on purchasing power parity.

Ecola et al., “The Future of Driving in Developing Countries.”

Ecola et al., “The Future of Driving in Developing Countries.”

Knoema, “Tonga GDP per Capita, 1980-2017,” accessed March 19, 2018, https://knoema.com//atlas/Tonga/GDP-per-capita.

New Zealand Ministry of Transport, “Transport Volume : Vehicle Travel,” accessed March 19, 2018, http://www.transport.govt.nz/ourwork/tmif/transport-volume/tv002/.

Include citation ”Generated Traffic and Induced Travel: Implications for Transport Planning,” (Todd Litman, Victoria Transport Policy Institute, April 2018) www.vtpi.org/gentraf.pdf

“Vehicle Fuel Efficiency (CAFE) Requirements by Year” (U.S. Department of Energy: Office of Energy Efficiency and Renewable Energy), www.afdc.energy.gov/data/10562

“Idling Reduction Savings Calculator, Backgroud Data”. Argonne National Laboratory www.anl.gov/sites/anl.gov/files/idling_worksheet.pdf

Argonne National Laboratory, Idele Base. http://cleancities.energy.gov/idlebase

“Idling Reduction for Long-Haul Trucks: An Economic Comparison of On-Board and Wayside Technologies,” (Argonne National Laboratory, September 2016) https://anl.app.box.com/s/ordxowhhcqrp7fe3yd3oy2fqpwt3fpiw

Assuming that each person produces approximately 22 lbs. of waste grease per year. www.nrel.gov/docs/fy99osti/26141.pdf

Winfried Scheewe, “Coconut Farm Diversification,” Group of Advocates for Sustainable Agriculture Inc. 2003. https://c.ymcdn.com/sites/echocommunity.site-ym.com/resource/collection/f6ffa3bf-02ef-4fe3-b180-f391c063e31a/coconut_farm_diversification_gasa__engl2.pdf

“Overview of the DOE VTO Advanced Battery R&D Program,” (U.S. Department of Energy: Office of Energy Efficiency and Renewable Energy, June 2016). https://energy.gov/sites/prod/files/2016/06/f32/es000_howell_2016_o_web.pdf

“CAIT Country Greenhouse Gas Emissions: Sources & Methods” (World Resources Institute, June 2015) http://cait2.wri.org/docs/CAIT2.0_CountryGHG_Methods.pdf

David C. Love et al., “Is the Three-Foot Bicycle Passing Law Working in Baltimore, Maryland?,” Accident Analysis & Prevention 48 (September 2012): 451–56, https://doi.org/10.1016/j.aap.2012.03.002.

“2016 Pedestrian Saftey Data,” (NHTSA, February 2018) https://crashstats.nhtsa.dot.gov/Api/Public/ViewPublication/812493.

Joanne M. Wood et al., “Using Reflective Clothing to Enhance the Conspicuity of Bicyclists at Night,” Accident Analysis & Prevention 45 (March 2012): 726–30, https://doi.org/10.1016/j.aap.2011.09.038.

“Compilation of State, County, and Local Anti-Idling Regulations” (Environmental Protection Agency, April, 2006) www.epa.gov/sites/production/files/documents/CompilationofStateIdlingRegulations.pdf

“Biodiesel Benefits and Considerations” (U.S. Department of Energy: Office of Energy Efficiency and Renewable Energy) https://www.afdc.energy.gov/fuels/biodiesel_benefits.html

Cooperative Extension System. “Extension Economics of Small-Scale Biodiesel Production,” March 2012. http://articles.extension.org/pages/30024/economics-of-small-scale-biodiesel-production

Hanna Kalenoja, “Energy Consumption and Environmental Effects of Passenger Transport Modes––A Life Cycle Study on Passenger Transport Modes,” Tampere University of Technology. 1996. Page 15.

Tebbutt Research, “Survey of Consumer Awareness and Use of Energy Rating Labels in PICs: Tonga Country Report” (The Pacific Commuity)

“Kingdom of Tonga: Electric Supply System Load Forecast” (Asia Sustainable and Alternative Energy Program, March 2010), 19, https://www.astae.net/sites/astae/files/publication/Tonga-Electric-Supply-System-Forecasts%5B1%5D.pdf.

www.globalfueleconomy.org/transport/gfei/autotool/approaches/economic_instruments/varied_registration_fees.asp

https://www.nrel.gov/transportation/drive.html